# Circuit Art

| Subject: Electricity, Circuits Related Subjects: Electrical Engineering | Grade Level(s): 4–8 Length of Lesson: 60 minute | Type: Project Keywords: Electricity, Circuits, LEDs, Conductivity, Series Circuit |
| --- | --- | --- |

# Lesson Overview

In this engaging activity, students will create a light-up art board using LEDs and aluminum foil to build a working series circuit. Students will explore how electrical current flows through a closed loop and how artistic design can be integrated with engineering principles.

# Lesson Focus

Students will construct a display board with numerous LED lights.

| # Lesson Objective(s) | - Students will construct a series circuit. - Students will demonstrate electricity flowing. |
| --- | --- |

# Lesson Timing

| 10 minutes | Introduction to circuits and materials |
| --- | --- |
| 30 minutes | Board design and construction |
| 10 minutes | Testing and troubleshooting |
| 10 minutes | Discussion and reflection |

| # Materials | - Aluminum foil - LED lights - Paper clip - Tape - Push Pin - Fasteners - 2 AA battery holder - Sandwich toothpick - Foam Board - Batteries |
| --- | --- |
| # Instructor Prep | - Pre Cut foam boards and distribute components in kits. - Prepare visuals showing series vs. parallel circuits. - Demonstrate how to identify LED terminals (long = positive, short = negative). - Set up a testing station with extra batteries and replacement parts. |
| # Related Resources |  |

# Lesson Plan

## Introduction

Introduce the concept of electrical circuits and how electricity flows from a power source through conductors and back. Explain the roles of components such as LEDs, batteries, fasteners, and switches. Emphasize the creative aspect—students are designing artwork that lights up!

## Procedure

- Cut 5”x5” piece of foam board.

- Poke one hole  2” from top and ½ “from side.

- Poke another hole 1” below first.

- Using pushpin, create a pattern for where the LED lights will go (two holes for each light).

- Poke hole on back of board near top.

- Insert fasteners into holes on the side.

- Insert LED lights.

- Use foil to connect all the legs of the lights, make sure to separate long legs from short legs, tape into place.

- Strip ends of wires on battery holder.

- Glue battery holder to back of board near bottom.

- Attach black wire to leg of fastener closest to battery holder

- Attach red wire to foil connecting long legs.

- Attach foil from remaining fastener to foil connecting long legs.

- Glue sandwich toothpick to hole on top of board.

- Attach a paper clip to one fastener.

- Insert batteries and complete circuit by touching paper clip to other fastener

## Wrap-up

Discuss:

- What made some circuits work and others not?

- Why is it important to separate the long and short legs of LEDs?

- How does aluminum foil act as a conductor?

- How would you improve or redesign your board?

Encourage creativity in future versions, such as spelling names or drawing symbols using LEDs.

# Lesson Background for Teachers

This activity teaches students about electric current, series circuits, and design thinking. It reinforces the need for clear paths, directionality of current, and problem-solving when circuits don’t work on the first try.

## Lesson Variations and Options

- Have students build parallel circuits to compare brightness.

- Use copper tape instead of foil for cleaner lines.

- Add a theme: create light-up holiday cards or science fair logos.

## Explanation

Electricity flows through a closed circuit. If the path is interrupted, the current stops. LEDs are directional—they only allow current to flow one way. This activity also simulates how real switches control flow and how conductive materials (like foil) transmit energy.

## Key Concepts and Vocabulary

- Series Circuit: A circuit with one path for electricity to follow

- Conductor: Material that allows electricity to flow through it

- LED (Light Emitting Diode): A component that lights up when electricity passes through

- Switch: A device used to open or close a circuit

- Current: The flow of electric charge

## Safety Notes

- Do not short circuit the battery holder.

- Supervise use of pushpins and sharp tools.