# Kaleidoscope

| Subject: Light, Reflection, Measurement – Physics, Optics Related Subjects: | Grade Level(s):k,2,3,4,5,7 Length of Lesson: | Type: Inquiry / Design / Project Keywords: Light, reflection, measurement |
| --- | --- | --- |

# Lesson Overview

Students will use bottles and reflecting surfaces to create a device that turns ordinary colored

objects into beautiful array of images.

# Lesson Focus

The main phenomenon of this activity is to notice and see patterns from the bouncing reflections of different mirrors.

| # Lesson Objective(s) | • Students will create a kaleidoscope. • Students will demonstrate the effects of reflection |
| --- | --- |

# Lesson Timing

| 5 minutes | Introduce the Lesson |
| --- | --- |
| 30 minutes | ... |

| # Materials | - •–Tube 5cm x 27cm or plastic water bottle - • Mylar - • Foam board - • Xacto knife - • Black electrical tape - • Scissors - • Beads or Marbles - • Large clear plastic cups - • Glue |
| --- | --- |
| # Instructor Prep | - Step 1 |
| # Related Resources | - https://www.wikihow.com/Make-a-Kaleidoscope - https://babbledabbledo.com/mini-diy-kaleidoscopes-open-ended/ |

# Lesson Plan

## Introduction

The first kaleidoscope was made from a tube containing loose pieces of colored glass and other

objects that were reflected by mirrors set at angles. This created beautiful patterns that could be

seen when looking through the end of the tube.

The kaleidoscope was invented by Sir David Brewster, a Scottish scientist, in 1816. The name comes from the Greek words, kalos (beautiful), eidos (form) and scopos (watcher). The first kaleidoscope was made from a  tube containing loose pieces of colored glass and other objects that were  reflected by mirrors set at angles.  This created beautiful patterns that could be seen when looking through the end of the tube.  American Charles Bush made improvements in kaleidoscopes in the 1870s.

Primer Questions:

- What are applications of light?

  - Lighting

  - Lasers

  - *Variety of other answers

- What are some things we know about light?

  - They warm things up

  - It’s a form of energy.

## Procedure

- –Cut 3 strips of foam board 4cm in width and 15cm in length each

- –Cut 3 strips of Mylar paper 4cm in width and 15cm in length each

- –Cut off the bottom of the water bottle

- –Wrap black electric tape around the entire body

- –Carefully cover a foam board strip with a thin layer of white glue (not hot glue)

- –Gently and carefully lay the Mylar paper on top of the foam board strip.

- –Repeat for the other 2 foam board and Mylar strips

–Create a triangle with the foam board strips with the Mylar sides facing inward.

- –Wrap paper around triangle so that it is a snug fits and insert triangle into water bottle tube until it is flush at the end.

- –The mouth of the bottle is your eye piece

- –Firmly insert the end of bottle into one plastic cup

- –Add 1 layer of beads or marbles to the other plastic cup

- –Place cup end of tube into cup with beads or marbles.

- –Look through and turn.

# Instructions for tube

- –Cut 3 strips of foam board 4cm x 24cm each

- –Cut 3 strips of Mylar paper 4cm x 24cm each

- –Carefully cover a foam board strip with a thin layer of white glue (not hot glue)

- –Gently and carefully lay the Mylar paper on top of the foam board strip.

- –Repeat for the other 2 foam board and Mylar strips

–Create a triangle with the foam board strips with the Mylar sides facing inward.

- –Insert triangle into cardboard tube until it is flush at one end. The other end will be recessed in about 2cm

- –Cut a hole 1.5cm diameter centered in the bottom of a film canister. This  is your eye piece

- –Using hot glue, glue the bottom of film canister (with the hole centered over the hole in the triangle, to the triangle recessed inside of the tube.

- –Firmly insert other end of tube into one plastic cup

- –Add 1 layer of beads or marbles to the other plastic cup

- –Place cup end of tube into cup with beads or marbles.

- –Look through and turn.

## Wrap-up

Student uses different color marbles and/or beads to get different patterns.

# Lesson Background for Teachers

## Suggested Real-World STEM Connections

Yayoi Kusama

Born in 1929 in Matsumoto, Japan, Kusama’s work has been featured widely in both solo and group exhibitions. She presented her first solo show in Japan in 1952. In the mid-1960s, she lived in New York where she became an important avant-garde artist by staging groundbreaking and influential happenings, events, and exhibitions. In 1965 Kusama produced her first Infinity Mirror Room. Building on the repetition found in her paintings and sculptures, this artwork used mirrors to create an illusion of infinite space. Since then Kusama has made more than 20 unique rooms.

# Real World Example

- Lasers

- Ovens

- GPS and Navigation

- Solar Panels

# Hypothesis & Prediction

- Students will predict the behavior of light when different mediums are placing between the observer and the opening of the cone

# Brainstorm & Design

- Students will attempt to design a unique pattern based off what they know of light

- Students will use the above materials to develop a kaleidoscope

- How will they overcome those constraints?

# Build & Create

- As above, students will use all of the materials above with a bit of general guidance in order to create their own kaleidoscopes.

# Test & Record

- Students will view through their own created kaleidoscopes and record observations

# Assess & Evaluate

- Students will make detailed observations as well as recognize the changes that different colors make to the kaleidoscope patterns.

# How to Improve?

- Ask the students what they would improve or change if they were to do it again

## Lesson Variations and Options

Different numbers and shapes of mirror/reflective surface  alignment can be tried to observe the variation and complexity in patterns created by the mirror and the complex mirror orientation in the tube.

## Explanation

When three mirrors of the same size are arranged into an equilateral triangle, rays of light will reflect to form multiple images of any object placed between the mirrors. This is how a kaleidoscope works. The patterns you see through a kaleidoscope are made by light bouncing between the mirrors. When you look through one end of a kaleidoscope, light enters the other end and reflects images of various objects off the mirrors, creating amazing patterns.

Light: Light is a wave, similar to the waves in the ocean. The difference is that light waves are very small. Light waves are all around us, and they interact with every object they encounter. It is light waves that create our perception of colors. To see color, you have to have light. When light shines on an object some colors bounce off the object and others are absorbed by it. Our eyes only see the colors that are bounced off or reflected.

The sun’s rays, and our overhead lights, contain all the colors of the rainbow mixed together. This mixture is known as white light. When white light strikes a white object, it appears white to us because it absorbs no color and reflects all color equally. A black object absorbs all colors equally and reflects none, so it looks black to us. While artists consider black a color, scientists do not because black is the absence of all color.

Mirrors/reflection: A mirror is a shiny surface that can reflect light so well that it preserves any image placed in front of it. This is because a mirror is very smooth, so light reflects off of it evenly. If there is any roughness to a surface, that roughness will cause light to scatter, which distorts an image. What you are actually seeing, when you look in a mirror, is light reflecting off of you! In the daytime, or under artificial lights, light reflects off your body in all directions. That's why you can see yourself and other people can see you. Your skin and the clothes you're wearing reflect light in a diffuse way: light rays bounce off randomly, haphazardly, in no particular direction. Stand in front of a mirror and some of this light from your body will stream in straight lines toward it. This light is reflected directly back off the smooth mirror, and you see your image.

## Key Concepts and Vocabulary

- Tube: The main body of the kaleidoscope, typically cylindrical.

- Mirrors: Inside the tube, usually arranged in a triangular configuration. The number and arrangement of mirrors determine the complexity and type of patterns.

- Object Chamber: A container at one end of the tube holding small, colorful objects like beads, glass, or other trinkets.

- Lens: Sometimes placed at the end opposite the object chamber to focus the viewer's eye.

- Eyepiece: The end of the tube where the user looks through.

## Safety Notes

- Students should use caution with sharp tools

- Students should use caution with glue guns