# Theodolite to find the Height

| Subject: Architecture, Trigonometry Related Subjects:Measurement, Distance, Height | Grade Level(s): 2-7 Length of Lesson: | Type: Inquiry / Design / Project Keywords:Angle, Protractor, Feet, Degrees, Acute, Right, Obtuse, Tangent |
| --- | --- | --- |

# Lesson Overview

# The theodolite is an instrument that is used in surveying. It is used to measure the angles of remote objects with respect to a flat plane, such as the ground. The evolution of the theodolite began with the description of the instrument included within the book Pantometria, by English mathematician Leonard Digges, which was published posthumously by his son Thomas in 1571.

# Lesson Focus

Students will create a Theodolite and use it to make height measurements for various objects.

| # Lesson Objective(s) | - Students will construct a Theodolite - Students will take measurements of the height of certain objects using Theodolite |
| --- | --- |

# Lesson Timing

| 5 minutes | Introduce the Lesson |
| --- | --- |
| 30 minutes | ... |

| # Materials | - –1/2 inch PVC pipe - –Film canister - –Plastic protractor - –1/8th inch wood dowel - –Alligator clip - –String - –1 ½ inch x 3/16th inch bolt - –3/16th inch drill bit - –1/8th inch drill bit - –Drill - –Hot Glue gun - –Glue sticks - –PVC pipe insulator - –Empty toilet paper roll - –Tangent angle table - –Yard stick - –Scissors - –Stapler |
| --- | --- |
| # Instructor Prep | ANGLE; FEET; ANGLE; FEET; ANGLE; FEET. 1; 0.175; 31; 6.009; 61; 18.04. 2; 0.349; 32; 6.249; 62; 18.807. 3; 0.524; 33; 6.494; 63; 19.626. 4; 0.699; 34; 6.745; 64; 20.503. 5; 0.875; 35; 7.002; 65; 21.445. 6; 1.051; 36; 7.265; 66; 22.46. 7; 1.228; 37; 7.536; 67; 23.559. 8; 1.405; 38; 7.813; 68; 24.751. 9; 1.584; 39; 8.098; 69; 26.051. 10; 1.763; 40; 8.391; 70; 27.475. 11; 1.944; 41; 8.693; 71; 29.042. 12; 2.126; 42; 9.004; 72; 30.777. 13; 2.309; 43; 9.325; 73; 32.709. 14; 2.493; 44; 9.657; 74; 34.874. 15; 2.679; 45; 10; 75; 37.321. 16; 2.867; 46; 10.355; 76; 40.108. 17; 3.057; 47; 10.724; 77; 43.315. 18; 3.249; 48; 11.106; 78; 47.046. 19; 3.443; 49; 11.504; 79; 51.446. 20; 3.64; 50; 11.918; 80; 56.713. 21; 3.839; 51; 12.349; 81; 63.138. 22; 4.04; 52; 12.799; 82; 71.154. 23; 4.245; 53; 13.27; 83; 81.443. 24; 4.452; 54; 13.764; 84; 95.144. 25; 4.663; 55; 14.281; 85; 114.301. 26; 4.877; 56; 14.826; 86; 143.007. 27; 5.095; 57; 15.399; 87; 190.811. 28; 5.317; 58; 16.003; 88; 286.363. 29; 5.543; 59; 16.643; 89; 572.9. 30; 5.774; 60; 17.321; 90; Orbit. |
| # Related Resources | - List related lessons/ppts etc available from USC or outside, trusted orgs |

# Lesson Plan

## Introduction

When two lines connect or cross, they form an angle.  Angles are measured in terms of degrees. Two form a right angle when they connect or cross with one going up and down and the other going right to left.

# A right angle equals 90 degrees.  An angle that is less than 90 degrees is called an acute angle.  An angle more than 90 degrees is called an obtuse angle. A triangle is a figure with 3 angles and three sides.  All of the angles of a triangle will add up to 180 degrees. A right triangle has at least right angle and it is usually shown with a square.

The angles and sides of a right triangle have special relationships.  One of these relationships is called the tangent.  The tangent of one of those angle is equal to the side opposite of the angle divided by the side next to the angle.

1

2

a

The tangent of angle a is equal to side 1 divided by side 2.  To measure heights, we only use acute angles. We have charts which show the tangent for angles from 0 to 89.  We can use these charts to figure out height.

The height of side 1 is equal to the tangent of a times the length of side 2.  If we make side 2 10, the height of side 1 is 10 times the tangent of angle a.  In our Theodolite, we make the string 10 ft.  We use the protractor to find the angle.  Using them together plus a tangent chart, we created the spinning length finder. So, you can always find the height of any object that you stand 10 feet from.

## Procedure

- –Cut a 7 ½ inch piece of PVC

- –Cut a 9 ½ inch piece of PVC

- –Drill (with 3/16th inch bit) a hole ½ inch from the end of the longer PVC completely through both sides

- –Drill (with 3/16th inch bit) a hole 1 ½” from end of shorter PVC through one side

- –Gently drill (with 3/16th inch bit) a hole through the bottom center of the protractor

- –Drill (with 3/16th inch bit) hole through center of bottom of film canister

- –Screw bolt through holes (right to left) Bottom of film canister, longer PVC, protractor and into shorter PVC.  The

The end of the shorter PVC should extend 1 inch beyond the longer PVC.  The protractor should be aligned with the curved in pointing toward the ends of the PVC with the bolt

- –Glue protractor to longer PVC to keep it from moving

- –Adjust the bolt so the PVC and film canister can rotate easily, but stay in place once moved.

- –Tie one end of string to alligator clip

- –Measure and cut string so that string and alligator clip together equal 10ft ½in

- –Staple free end of string to side of film canister and tie off string so that remaining string and alligator clip together equal 10ft

- –Wind string around film canister

- –Using 1/8th inch drill bit, drill hole in shorter PVC on one side (this is now the top) so that:

- the hole is lined up with and perpendicular to the bolt

- the hole is lined up with the protractor so that when you insert a dowel it will line up with the zero degree mark on the protractor

- – Using 1/8th inch drill bit, drill a on top of the longer PVC ¼” from far end (without bolt) and 5” from far end

- –Cut 3- 1 ½” dowels

- –Make mark one dowel.  Insert and glue in hole near protractor so mark can be seen and lines up with 0 degree mark

- –Cut 4” of PVC insulator and slide on far end of longer PVC (you may need to adjust bolt) between two holes

- –Cut out Tangent Table at end of this lesson and tape around toilet roll (you may need to adjust size of table to make it fit)

- –Slide toilet roll around PVC insulator

- –Insert and glue dowels to hold toilet roll in place.

- –Measure the height of your eyes from the ground

- –To use Theodolite, attach alligator clip to an object and extend 10ft of string.

- –Aim longer PVC along string line at object.

- –Look through shorter PVC

- –While keeping longer PVC level, move shorter PVC until you see the top of the object.

- –Look to see what number (angle) on the protractor that the marking on the dowel is even with.

- –Rotate toilet paper roll to find the angle.

- –To the right of the number will be the distance.

- –Add this number to the height of your eyes.

- –This will be the approximate height of the object.

## Wrap-up

- Students will calibrate Theolodite on objects with known height such as a basketball rim.

# Lesson Background for Teachers

# Safety Procedures

- Students should exercise caution when using hot glue gun

- Students should exercise caution when using scissors

- Students should exercise caution when using saw

- Students should exercise caution when using drill

- Students should NEVER look at the sun with the Theodolite

- Students should exercise caution with alligator clips