# Mousetrap Car

| Subject: Physics, Motion and Forces Related Subjects: Mechanical Engineering | Grade Level(s): All grades Length of Lesson: | Type: Project Keywords: Kinetic Energy, Potential Energy, Motion, friction, Inertia |
| --- | --- | --- |

# Lesson Overview

Create a vehicle powered by a standard-sized mousetrap to travel the furthest distance.

# Lesson Focus

How can we harness stored energy in a mousetrap to create motion and propel a vehicle forward? Students will learn about how potential energy is converted to kinetic energy, making cars move forward.

| # Lesson Objective(s) | By the end of this lesson, students will learn that the - Understand how potential energy is stored and converted into kinetic energy. - Investigate the role of friction, rotational motion, and mechanical advantage in motion. - Use the engineering design process to test and improve a mousetrap-powered vehicle. - Collaborate and apply problem-solving skills to optimize their vehicle's performance. |
| --- | --- |

# Lesson Timing

| 5 minutes | Introduce the Lesson |
| --- | --- |
| 30 minutes | Building the cars in teams. |
| 15 minutes | Testing, modifying, and competing for the longest distance. |
| 10 minutes | Wrap-up discussion |

| # Materials | - Mousetrap - Wood dowels - Plastic bottle tops - Hot glue - Eyelets - Electrical tape - Purple Plastic wheels - String - CDs - Scissors - Straws |
| --- | --- |
| # Instructor Prep | - Pre-cut dowels and pre-poke bottle cap centers if working with younger students. - Review safety precautions for handling mouse traps and hot glue. - Set up a long test track (hallway or straight classroom floor space). - Have extra materials ready for troubleshooting. |
| # Related Resources | - Mousetrap Car - Explained : 11 Steps (with Pictures) - Instructables |

| Troubleshooting |  |
| --- | --- |
| If the car is… | Then… |
| Not moving at all | · The wheels are pressed too tightly against the straw · There’s excessive hot glue or something else interfering with the wheel rotation · The mousetrap arm is too long |
| Not driving straight | · The wheel axles are not parallel to each other · One of the small wheels is moving but the other isn’t |
| Not coasting very far | · There’s a source of excessive friction affecting the wheels · The car is too heavy |

# Lesson Plan

## Introduction

- The mousetrap stores potential energy when the spring-arm is pulled back.

- The string is wound around the drive wheel axle.

- When released, that potential energy is converted into kinetic (moving) energy in the form of the drive wheels turning.

- The distance the metal spring-arm travels is short, but the energy output is very strong.

Through the extended arm and large wheels, that short burst of energy from the mousetrap is converted into a slow, longer-lasting output of energy because it has a very low mechanical advantage.

## Procedure

- Glue bottle top to center of a CD.

- Poke a hole in the center of the bottle top.

- Repeat for 2nd CD.

- Cover CDs with latex gloves for traction – these will be your rear wheels.

- Use the thicker wood dowels to form the chassis (body) of the car. Typical shapes:

- Use the smaller dowels for the axles.  You can use straws or the eyelets to hold the axles and allow them to turn.

- Glue the mousetrap to the chassis.

- Attach a lever arm to the mousetrap by using electrical tape.

- Tie a string to one end of the lever arm.

- Wind string around rear axle.

## Wrap-up

The car will move forward as long as the mousetrap arm pulls on the string, which in turn rotates the drive axle. Once this action stops, the car will continue to move forward due to its momentum. However, friction between the straws and the axles will gradually slow the car down. To increase the coasting distance, friction should be minimized. This can be achieved by removing excessive hot glue, ensuring the wheels do not touch the straws, and reducing the contact area between the dowels and the straws. Additionally, making the car lighter by removing unnecessary material can further reduce friction.

# Lesson Background for Teachers

## Suggested Real-World STEM Connections

Dr. Gladys West, born in Sutherland, VA, grew up in Dinwiddie County and was one of four children. Determined to avoid field work like her parents, she excelled in education, becoming the valedictorian of her high school. She earned a BS and Master’s in Mathematics from Virginia State University, a Master’s in Public Administration from the University of Oklahoma, and a PhD in Public Administration from Virginia Tech.

In 1956, Dr. West began her 42-year career at the Naval Proving Ground, now Naval Support Facility Dahlgren, where she was the second African American woman hired. She played a crucial role in developing GPS technology, earning commendations for her work on satellite geodesy and radar altimetry data processing. She published numerous papers and presented internationally before retiring in 1998.

Recognized as a "hidden figure," Dr. West has received global accolades, including being named one of the top 100 Women by the BBC and inducted into the Air Force Missile and Space Pioneers Hall of Fame. At 90, she remains active in her community and church. Her memoir, "It Began with A Dream," was released on Juneteenth.

Dr. Gladys West is a legend, an inspiration, and a true American “Shero.”

## Lesson Variations and Options

Length string/arm/frame, weight of the frame - The basic example of a mousetrap car provided is a good starting point, but it's not the ultimate design. We can leverage our understanding of the mathematics and science behind the car to explore ways to enhance its performance.

The optimal mousetrap car begins by moving forward slowly, utilizing minimal energy to start. This indicates the lowest possible mechanical advantage. As the car progresses, it builds momentum. By the time the arm reaches the end of its arc, the car should have enough momentum to continue coasting for some distance. The less friction the car generates, the farther it will coast.

To optimize the car, students should consider the following categories:

Weight

Optimization: The most straightforward improvement is to reduce the car's weight. A lighter car generates less friction, allowing it to coast further.

Questions:

- What materials can be removed without compromising the car's functionality?

- Can lighter materials replace the heavier ones?

Friction

Challenge: Friction is the biggest hindrance to a high-performing mousetrap car. There are ways to reduce friction without necessarily reducing weight.

Steps:

Identify and correct imperfections causing excessive friction, such as bits of hot glue inside the straws or wheels pressed too tightly against the straws. Small adjustments can significantly improve performance.

Questions:

- What additional steps can be taken to reduce friction?

- How can the design be modified to prevent the wheels from rubbing against the straws?

- How can contact between the front wheel dowel and the straw be minimized?

String Length

Consideration: The length of the string, as well as the extended arm and frame, determines how many times the drive wheels rotate under the mousetrap's power. However, an excessively long string increases weight and friction.

Issues:

A string that is too long may create an extremely low mechanical advantage, making the output force too weak to overcome the car's inertia.

Questions:

- How long can the arm be before it becomes impractically long?

Drive Wheel Dowel

Experimentation: Lowering the mechanical advantage can also be achieved by changing the drive wheel dowel thickness. Thinner dowels can be used and made to fit the wheels by wrapping tape around the ends.

Example: Replacing a 0.25" (1/4") dowel with a 0.125" (1/8") dowel theoretically increases the rotation to approximately 80 feet.

Practical Issues: In practice, a thin dowel may bend under strain, causing excessive friction or preventing movement. Also, wrapping the string evenly can be challenging.

Questions:

- What is the ideal dowel thickness?

- Are there more efficient string-like materials for wrapping around the dowel?

- By exploring these areas, students can develop a deeper understanding of how to enhance their mousetrap car's performance.

## Explanation

Mechanical Advantage

- The mousetrap car uses mechanical advantage to convert the quick burst of energy from the spring into a weaker but longer-lasting output. The mechanical advantage can be calculated as follows:

- The metal spring-arm of the mousetrap is 2 inches long, and the craft stick mousetrap arm is 12 inches long, giving a mechanical advantage of 2/12 or 1/6.

- This means the force at the end of the craft stick arm is 6 times weaker than the force at the metal spring-arm, but it travels 6 times farther.

- The drive wheel dowel has a diameter of 0.25 inches, and the wheel diameter is 4.75 inches, resulting in a mechanical advantage of 0.25/4.75 or 1/19.

- The total mechanical advantage is calculated by multiplying the two: 1/6 x 1/19 = 1/114.

- This means the force at the drive wheels is 114 times weaker than at the metal spring-arm but travels 114 times farther.

- A low mechanical advantage (1/114) is beneficial for mousetrap cars as it uses the energy from the mousetrap over a longer period, allowing the car to travel farther.

- In contrast, a high mechanical advantage would be useful for lifting heavy objects but would require a longer distance to apply the force.

Calculating Minimum Distance

- The minimum distance can be calculated as follows:

- The circumference of the dowel axle is π x 0.25 = 0.785 inches.

- The string length is about 25 inches.

- Therefore, the string can wrap around the dowel 25/0.785 = approximately 31.8 times.

- The circumference of the wheels is π x 4.75 = 14.92 inches.

- So, the minimum distance traveled will be 31.8 x 14.92 = 474.456 inches, or about 39.5 feet.

- Additionally, the car will continue to travel due to momentum, which depends on factors like car weight, surface type, and axle friction, making precise calculation difficult.

Arm Length

- A longer string results in a lower mechanical advantage, making the drive wheels turn more times and the car travels farther.

- However, a longer string also requires a longer arm and frame, adding weight.

- If the frame is shorter than the string, part of the string won’t wrap around the drive wheels, wasting its potential.

- Additional weight generates more friction, particularly between the straws and the dowels.

- Therefore, the string should be long enough to create a low mechanical advantage but not so long that it necessitates a heavy, slow car.

Wheel Size

- Larger wheels theoretically increase the distance traveled as a larger circumference means a greater minimum distance.

- The formula is (drive wheel circumference) x (number of times the string is wrapped around the dowel).

- However, larger wheels can be heavier, increasing friction where the wheel contacts the ground.

- Heavier wheels also have more inertia, requiring more energy to move.

- Energy transfer losses mean larger wheels cannot store extra energy efficiently as momentum.

- The balance is to minimize friction and inertia while decreasing mechanical advantage as much as possible.

Summary of Formulas: Mechanical advantage = (length of metal spring-arm / length of extended mousetrap arm) x (circumference of axle dowel / circumference of drive wheel)

Minimum travel distance = (string length / circumference of axle dowel) x circumference of drive wheel

## Key Concepts and Vocabulary

- Energy conversion is when one kind of energy is changed into another kind

- Mechanical advantage is the ratio of the input force to the output force

- Friction is a force that opposes the motion between two things that are in contact

## Safety Notes

- Mousetraps are only unsafe when the metal spring-arm is pulled all the way back. Pulling it back slightly while constructing the mousetrap arm is not particularly dangerous.