# Paper Cup Motor

| Subject: Circuit & Circuit Design Related Subjects: Aerospace Engineering, Electrical Engineering | Grade Level(s): 5-8 Length of Lesson: 90 minutes | Type: Design Keywords: Magnet, pole, polarity, electromagnet, current, induction |
| --- | --- | --- |

# Lesson Overview

Electric motors are part of many everyday devices, from electric cars to ceiling fans. These motors operate using the principles of magnetism and electromagnetism. In this lesson, students will construct a working motor using simple materials like a paper cup, magnet wire, and permanent magnets. This project introduces key concepts in electromagnetism and demonstrates how electricity can create motion.

# Lesson Focus

How can electricity and magnetism be used to create motion? Can students build a simple electric motor using everyday materials?

| # Lesson Objective(s) | - Understand that electricity in a coil of wire creates an electromagnet. - Demonstrate the interaction of electromagnets and permanent magnets. - Build a functional electric motor using a paper cup and other common materials. - Explain why the motor spins and how design changes affect its performance |
| --- | --- |

# Lesson Timing

| 10 minutes | Introduction to electromagnetism |
| --- | --- |
| 15 minutes | Teacher demonstration (coils and magnets) |
| 60 minutes | Student construction and experimentation with paper cup motor |
| 10 minutes | Wrap-up and reflection |

| # Materials | - Paper cups - Masking tape - Magnet wire - Permanent magnets (1”x.75” rectangle) - Sandpaper - Wire cutters - Batteries - Aluminum foil - Insulated wire - Paper clips - Film canisters - Cardboard - Glue gun and glue sticks |
| --- | --- |
| # Instructor Prep | - Pre-cut wires and prepare sandpaper stations - Set up demonstration area with nails, paperclips, and different batteries - Assemble an example motor ahead of time |
| # Related Resources | - Get Your Motor Running - Activity - TeachEngineering |

# Lesson Plan

## Introduction

Electric motors are a large part of our lives; from refrigerators, to electric cars, to fans, to elevators.   To make them work, we rely on the attraction and repulsion of magnets and the uses of electromagnets.

Permanent magnets have two poles, North and South (or charges, + and -).  The north pole of a magnet is attracted to the south pole and repels the north pole of another magnet.  The south pole of a magnet is attracted to the north pole and repels the south pole of another magnet.

Ampere’s circuital law shows that current flowing in a coil of wire makes the coil of wire a magnet (an electromagnet to be exact).  The more coils the wire has, the stronger the magnet.

When a battery is connected to the coil of wire, an electromagnet is formed and is either attracted to or repelled from the permanent magnet. This attraction or repulsion causes the coil to move.  We take advantage of this by positioning the coil so that it can spin freely above the permanent magnet and its momentum keeps it going.

## Procedure

Safety Procedures

- Students should use caution when using sandpaper and wire cutters

- Students should use caution when using glue gun

Instructions

Teacher demonstration only

- Cut 2 pieces of 2ft of magnet wire.

- Sand about ½” of ceramic coating from each end on both wires.

- Using one of the wires, make one big loop by twisting the ends of the wire (see Figure 2)

- Place a permanent magnet on the table.

- Hold the loop over the magnet about ½”.

- Connect battery and watch movement of loop toward or away from permanent magnet.  This is a weak electromagnet.

- Try to pick up paper clips with loop electromagnet.  It can’t!

- Wrap the other wire around a nail.

- Connect the battery and try to pick up a paper clip.  It works!

- Try to pick up a paper clip with another plain nail. Nothing!

- The only difference between the two electromagnets is the number of loops.  Hence, the more loops the stronger the electromagnet.

- Repeat steps 6 through 11 with stronger batteries.  Therefore, more current, stronger electromagnet

Components and Parts

- Cut a 2ft piece of magnet wire

- Sand about ½” of ceramic coating from each end.

- Leaving about 1 inch from the start and on the end, wrap the magnet wire around  (see Figure 2)

- Remove coil from film canister and wrap ends around coil once to hold its shape.

- Unbend bottom of two paper clips.

Construction

- Bend about 1cm of each in a right angle (see figure 3)

- Tape paper clips on cups, across from each other, with bent ends laying flat on the surface of the cup.  They should extend upward from the bottom of the cup (see figure 4).

- Drop one permanent magnet inside the cup.

- Place another permanent magnet on the bottom of the cup and hold until both magnets attract.

- Mount coil in paper clip cradle as shown in figure 1.

- Coil should be able to spin freely.

- Cut 2, 10” pieces of insulated wire.

- Strip both ends of both wires.

- Attach small pieces of aluminum foil to one end of each piece of wire.

- Attach other ends of wires to each paper clip cradle piece.

- Mount cup and wires to cardboard object with glue and tape.

- Squeeze aluminum foil ends on battery and watch spin.

- You may need to adjust the height or location of the paper clip cradle and/or  shape or position of the coil to get a spinning effect.

- To get it started, you may need to give it a gentle tap

Troubleshooting

- Ends of magnet wire may not be completely sanded

- Coil is uneven

- Connections from paper clip to wire may be loose

## Wrap-up

- Students should be able to explain why spinning occurs.

- Students should be able to troubleshoot and correct problems.

- Students should change variables and observer results

- Size or charge of battery

- Number of magnets

- Number of coils of wire

# Lesson Background for Teachers

This activity demonstrates Faraday's and Ampère's principles of electromagnetism. The coil spins due to the interaction between the electromagnetic field created by current and the field of the permanent magnet. When the coil's momentum breaks the circuit briefly, the spin continues, and the cycle repeats.

## Lesson Variations and Options

- Add a second battery to observe increased current effect

- Change coil size or number of loops

- Replace paper cup with different mounts

## Explanation

- Electromagnet: A magnet created when electric current flows through wire

- Polarity: Direction of magnetic force based on north/south poles

- Current: Flow of electric charge that powers electromagnetism

## Key Concepts and Vocabulary

- Magnet: An object that produces a magnetic field

- Pole: Either of two points where the magnetic force is strongest

- Electromagnet: A magnet created by electricity

- Current: The flow of electric charge

## Safety Notes

- Supervise use of sandpaper and wire cutters

- Use caution with glue guns to avoid burns