# Plinko

| Subject: elastic collisions , Probability, Gravity Related Subjects: | Grade Level(s): Length of Lesson: | Type: Inquiry / Design / Project Keywords: Random, elastic collision, path |
| --- | --- | --- |

# Lesson Overview

# Patterned after a game on a popular TV game show, Plinko is a fun way for kids to learn about gravity, and probability.

# Lesson Focus

Students will create a pattern of nails on an inclined board.  The pattern will allow multiple paths for a marble to roll.

| # Lesson Objective(s) | - Students will create a game using gravity as the force used to move a marble through a maze of nails. - Students will determine if a marble can consistently follow the same path through the plinko |
| --- | --- |

# Lesson Timing

| 5 minutes | Introduce the Lesson |
| --- | --- |
| 30 minutes | ... |

| # Materials | - –100 5/8” nails - –Hot Glue Gun - –Glue sticks - –Foam board or cardboard - –Rubber bands - –Plywood - –Saw - –Hammer - –paper cup - –Ruler - –Pencil |
| --- | --- |
| # Instructor Prep | - Step 1 |
| # Related Resources | - List related lessons/ppts etc available from USC or outside, trusted orgs |

# Lesson Plan

## Introduction

On a fixed area inclined board with a specific number of nails, there are a fixed number of paths that a marble can travel, given a specific starting point.  By simply rolling a marble on the board, the path it takes seems to be random.  However, its path is due to a number of elastic collision (collisions without deforming the object) that causes the direction of force, caused by gravity, to change.  There is an equal chance that the marble takes a particular path.  Using this phenomenon, we can create a fun game of chance.  We give varying points for the marble taking a particular path.

## Procedure

Components and Parts

- – Cut 4.5-inch x 9-inch (W x L) board.

Construction

- – Draw a grid on the wood with horizontal lines and vertical lines 1” apart

- – Hammer 5/8-inch nails in the corners of square (don’t hammer nails through board).

- – Glue 1” foam board (or cardboard) stop at bottom of plywood.

- – Glue cup to back of plywood at the top (creating an incline)

- – Wrap rubber band along nails on outside of each side of plywood (creating a side stop).

# Troubleshooting

- Make sure incline is large enough to make marble roll

- Make sure nails are spaced far enough for marble to roll through.

## Wrap-up

- Students should be able to explain probability.

- Students should be able to explain why marble goes in different directions.

# Lesson Background for Teachers

# Safety Procedures

- Students should use caution when using hammer and nails.

- Students should use caution when using hot glue gun.