# Spinners

| Subject: Angular Momentum ,Rotational Inertia Related Subjects: | Grade Level(s): 1,2,3,4,5,6,8 Length of Lesson: | Type: Inquiry / Design / Project Keywords:Diameter, rotate, inertia, momentum, friction |
| --- | --- | --- |

# Lesson Overview

Students will create three projects that utilize the physics of spinning to function.  They are the spinning top, buzz saw and spin master.  Each uses principles of rotation to function.

# Lesson Focus

What is the central question or phenomenon being investigated?  What problem are they trying to solve?

| # Lesson Objective(s) | - Students will create a spinning top - Students will create a buzz saw. - Students will create a African spinning toy. - Students will demonstrate various properties of rotating objects with their projects. |
| --- | --- |

# Lesson Timing

| 5 minutes | Introduce the Lesson |
| --- | --- |
| 30 minutes | ... |

| # Materials | - –Hole saw with ¼” guide - –Drill - –5/8” drill bit - –3/16” drill bit - –1/4” wood dowel - –1/2” pvc pipe - –Metal washer - –Pencil sharpener - –String - –Glue gun - –Hot glue sticks - –Scissors - –1/4” Wood boring drill bit - –Sander - –3x5 card |
| --- | --- |
| # Instructor Prep | - Step 1 |
| # Related Resources | - List related lessons/ppts etc available from USC or outside, trusted orgs |

# Lesson Plan

## Introduction

# Spinning objects are a regular part of our world.  In fact, our world itself is a spinning object. Ice skaters, Professional Car testers, Amusement park operators and nuclear engineers all use the physics of spinning objects in their respective careers.

## Procedure

Components and Parts

- –Drill 6 holes into wood about 4” apart.

- –Using hole saw, cut out 6 discs, using the predrilled holes as a guide.

- –Cut an 8” piece of ¼” wood dowel.

- –Cut 8” pvc pipe.

- –Cut 6”  ¼”wood dowel.

- –Cut 2- 2” pieces of wood dowel

Construction

### SPINNING TOP

- –Making sure the holes line up, hot glue two wood discs together

- –Sharpen one end of 8” dowel using a pencil sharpener.

- –Insert dowel through wood two discs until the pointed end sticks out by about 2”.

- –Hot glue dowel into place.

- –Spin top!

### Buzz saw

- –Making sure the holes line up, hot glue two wood discs together

- –Using 3/6” drill bit, make two holes on either side of center hole (about 1cm away) of discs.

- –Cut 24” string, insert through holes of disc and tie together

- –Cut four small triangles from 3x5 card and tape 90° apart.

- –Use 2- 2” pieces of wood dowel to hold both loop ends of string.

- –Wind and pull!

### African Spinning Toy

- –Making sure the holes line up, hot glue two wood discs together

- –Use boring drill bit to cut hole in pvc pipe ½” from top

- –Insert wood dowel into discs until flush with top and glue in place.

- –Glue washer to top of disc.

- –Insert dowel into pvc pipe.

- –Attach string to dowel through hole in pvc pipe.

- –Wrap string around dowel and pull!

# Troubleshooting

- Make sure strings are not tangled

## Wrap-up

- Students should give other examples of angular momentum.

- Students should be able to explain the similarities of all four projects.

# Lesson Background for Teachers

## Suggested Real-World STEM Connections

- List possible related Role Models in STEM (with details and links to more reading). Give suggestions for examples of scientists/engineers who are from underrepresented groups in STEM in related careers.

- List related real-world engineering problems and examples

## Lesson Variations and Options

Include descriptions of alternative methods or ways to present things.

## Explanation

Angular momentum keeps a spinning object spinning and also keeps it from tilting away from its orientation.  Rotational inertia also keeps the object spinning at nearly the same rate that it started (except for the effect of friction).  Each of the projects utilizes these physics properties.

## Key Concepts and Vocabulary

- Word 1: Grade Appropriate Definition

- Word 2: Grade Appropriate Definition

## Safety Notes

- Students should use caution when using drilling devices.

- Students should use caution when using hot glue gun.

- Students must ensure all clothing and hair are not loose of hanging.