# Toothpick Bridge

| Subject: Physics, Mathematics Related Subjects: Civil Engineering | Grade Level(s): 5–9 Length of Lesson: 60 minutes | Type: Project Keywords:Tension, Compression, Newton’s Third Law, Dynamic Loads |
| --- | --- | --- |

# Lesson Overview

When designing and constructing bridges, architects and civil engineers must be certain the bridges can handle a large amount of stress. The bridges must be designed with tension, compression, and other physical factors in mind so that it remains stable and meets design specifications and aesthetic standards.

# Lesson Focus

This activity will involve creating a bridge out of toothpicks so that students understand the basic standards architects and civil engineers must follow.

| # Lesson Objective(s) | - Students will design and construct a bridge from limited materials and the given bridge specifications. - Students will construct the bridge while acknowledging physical properties in play such as tension, dynamic loads, and compression so that the bridge can carry the maximum load. |
| --- | --- |

# Lesson Timing

| 5 minutes | Introduce the Lesson |
| --- | --- |
| 30 minutes | ... |

| # Materials | - □ Boxes of toothpicks - □ Roll of masking tape - □ Pennies or washers |
| --- | --- |
| # Instructor Prep | - Step 1 |
| # Related Resources | - List related lessons/ppts etc available from USC or outside, trusted orgs |

# Lesson Plan

## Introduction

Introduce students to the learning goals and make connections to past and present learning experiences. Stimulate interest and prompt students to identify their own questions about the topic. This can link the lesson to real world examples, introduce associated STEM careers, and assess their prior knowledge on the topic.

- Step 1

## Procedure

Bridge Specifications:

- Only at most 50% of a toothpick‘s total length surface may be taped. Both sides of each

stick can be considered in the 50 % calculation. e.g. 100% of side 1 and 0% of side 2;

75% of side 1 and 25% of side 2.

- The bridge must have a clear and unobstructed roadway at least 2 inches wide, running

the full length of the bridge, as if automobile traffic were going to cross it.

- Bridge can be any length possible

- Roadway must be at least 2 inches from the bottom of the bridge

- Only the number of pennies or washers that can fit on the bridge roadway will count.

Instructions:

- Students are given 60 toothpicks and 12 inches of masking tape and work in groups of 3-

4.

- Students will construct a bridge in pairs with roadway to hold the largest number of

pennies.

- Groups design and begin constructing their bridge while following the specifications

above.

- When construction is complete, begin placing pennies/washers to observe the maximum

for the bridge built.

- The winner is the team with the highest number of pennies/washers. In the case of a tie,

the bridge with the greatest amount of toothpicks remaining (least toothpicks used) wins.

## Wrap-up

End the lesson with some final discussions about what students designed/discovered. Connect their activity back to the focus question/problem, real world examples, and STEM careers.

Also include notes about any special cleanup considerations

# Lesson Background for Teachers

## Suggested Real-World STEM Connections

- List possible related Role Models in STEM (with details and links to more reading). Give suggestions for examples of scientists/engineers who are from underrepresented groups in STEM in related careers.

- List related real-world engineering problems and examples

## Lesson Variations and Options

Include descriptions of alternative methods or ways to present things.

## Explanation

Describe the background concepts and explanations behind the lesson.  This can be more in depth than a teacher/instructor would actually share with students.

## Key Concepts and Vocabulary

- Word 1: Grade Appropriate Definition

- Word 2: Grade Appropriate Definition

## Safety Notes

- Safety instruction