# Touch Switch Car

| Subject: Physics,Switches Related Subjects:Mechanical Engineering,Automotive Engineering | Grade Level(s):All Grades Length of Lesson: | Type: Project Keywords:Electronic, Civil Engineering,Circuits |
| --- | --- | --- |

# Lesson Overview

Touch switches are extremely useful in everyday life (touch screens use arrays of touch switches). Students will learn how important the ability to respond to the environment is. Previously they have made these exact same cars without these abilities and their cars would run into walls, stop, and/or break. Today, they will go home with a slightly improved version of that car that has greater functionality by avoiding getting stuck as often.

# Lesson Focus

What is the central question or phenomenon being investigated?  What problem are they trying to solve?

| # Lesson Objective(s) | By the end of this lesson, students will… - Learn the concepts of attraction, repulsion, friction, polarity, and charge. |
| --- | --- |

# Lesson Timing

| 5 minutes | Introduce the Lesson |
| --- | --- |
| 30 minutes | ... |

| # Materials | - Foam boards - Coffee stirrers - Wooden dowels (small for wheels, large for propeller mount) - Wheels - Straws - Glue guns - Glue slugs - Brass fasteners - Paper clips - Battery holders - Batteries - Wires - Motors - Propellers - Tape - Wire strippers |
| --- | --- |
| # Instructor Prep | - Step 1 |
| # Related Resources | - List related lessons/ppts etc available from USC or outside, trusted orgs |

# Lesson Plan

## Introduction

Introduce students to the learning goals and make connections to past and present learning experiences. Stimulate interest and prompt students to identify their own questions about the topic. This can link the lesson to real world examples, introduce associated STEM careers, and assess their prior knowledge on the topic.

- Step 1

## Procedure

List the steps of the activity.  Remember to focus on leading students to the solution, rather than just telling them what to do.  Include pictures of steps if you have them. Be sure to include some steps for discussion and reflection.

- The students will begin with the circuit

  - They will first draw the circuit, as it is the most complicated part of the project. The students will not build the circuit yet, as it will simply get in the way when building the car.

    - Using the circuit diagram, they must show that they understand that changing the current’s direction also changes the direction the motor spins. They can start by drawing simple circuit diagrams with just one battery and motor, and have them indicate the flow of electricity and the direction the motor spins.

    - Remind them of the alternative switches, we will build two of those.

- Start with the construction of the sensor part. Do not rush this part.

  - Adding brass fasteners to the foam board and paper clips to the “I” shaped movable sensor stick (made of three coffee stirrers |---| )

    - Brass fasteners must be evenly spaced in a line parallel to the direction of the car’s motion.

    - Make sure the paper clips, attached to the coffee stirrer are spaced so that they can come into contact with two non-adjacent brass fasteners at a time.

  - Add wires coming off of the bottom of each brass fastener when the fastener is placed on the car.

  - Add the small guiding sticks (placed to block the sensor piece from moving in undesired directions) when building this portion. Do not permanently attach the “I” bar to the foam board. It must be moveable.

  - Adjust the brass fasteners such that none of them touch each other and at any given time, only two of them are in contact with paper clips. You can use a coffee stirrer to make small adjustments by pushing underneath the top of the fastener; fingers are big such that they can accidentally misalign other parts.

- Create the circuit. Refer to the drawn circuit diagrams throughout this process.

  - Attach the two wires from the battery to the two paper clips.

  - Attach the two farthest brass fasteners to one side of the motor and the two inner brass fasteners to the other side of the motor. Make sure the motor is attached such that it will push the car away from stimuli.

- Mount the motor with its propeller. This can be done in any such matter (tape, glue). Adjust so that the car moves away from the wall that it sensed.

  - Make sure the propeller does not hit the car, ground, batteries, switch, or wires when it spins. Also make sure the propeller has airspace to push and pull from or the car will not effectively travel.

- Add the wheel and axles. Use the wooden dowels, straws, and glue.

  - Glue straws to the bottom of the car. Put appropriately sized wooden dowels in the straws. Attach wheels to the wooden dowels.

- Test it out, and most likely adjust the brass fasteners. Use little tools to help move the brass fasteners more precisely. Ideally, the brass fasteners should even be able to keep the paperclip somewhat steady when it is pushed to them. Both brass fasteners must have good contact with the paperclips for the car to work well.

## Wrap-up

End the lesson with some final discussions about what students designed/discovered. Connect their activity back to the focus question/problem, real world examples, and STEM careers.

Also include notes about any special cleanup considerations

# Lesson Background for Teachers

## Suggested Real-World STEM Connections

- List possible related Role Models in STEM (with details and links to more reading). Give suggestions for examples of scientists/engineers who are from underrepresented groups in STEM in related careers.

- List related real-world engineering problems and examples

## Lesson Variations and Options

Include descriptions of alternative methods or ways to present things.

## Explanation

Describe the background concepts and explanations behind the lesson.  This can be more in depth than a teacher/instructor would actually share with students.

## Key Concepts and Vocabulary

- Word 1: Grade Appropriate Definition

- Word 2: Grade Appropriate Definition

## Safety Notes

- Safety instruction