# Underwater ROV

| Subject: Related Subjects: | Grade Level(s): Length of Lesson: | Type: Inquiry / Design / Project Keywords: |
| --- | --- | --- |

# Lesson Overview

Engineering is the profession in which a knowledge of the mathematical and natural sciences, gained by study, experience, and practice, is applied with judgment to develop ways to utilize, economically, the materials and forces of nature for the benefit of mankind.

– Accreditation Board for Engineering and Technology

ENGINEERS TURN IDEAS INTO REALITY.  Engineers are problem-solvers, people who make things work better, more efficiently, quicker and less expensively.  They serve humanity with skill and dedication and search for better ways to solve problems. Engineers are in demand and are our future.  No matter your interest, there is an engineering field that can fit you.  Engineering fields include: aerospace, biomedical, chemical, civil, computer science, electrical, environmental, industrial, mechanical, nuclear, and petroleum.

# Lesson Focus

To build and test an underwater ROV which will be able to sample the fish that live on or near the bottom with an “otter trawl”.

| # Lesson Objective(s) | By the end of this lesson, students will… - Learn the concepts of attraction, repulsion, friction, polarity, and charge. |
| --- | --- |

# Lesson Timing

| 5 minutes | Introduce the Lesson |
| --- | --- |
| 30 minutes | ... |

| # Materials | - 2 – 8 ½” PVC pieces (top side edges) - 2 – 6” PVC pieces (front and center horizontal edges) - 2 – 5” PVC pieces (bottom side edges) - 4 – 4” PVC pieces (front and back vertical edges) - 6 – 2” PVC pieces (connectors between tees and elbow) - 10 PVC elbows - 4 PVC tees - PVC insulators - 7 bolts with nuts - 3 propellers - 3 motors - 3 bottle caps - 3 film canisters - 3 – DPDT switches - Wire stripper - Washers - Plastic container for switches - Screw driver - Ruler - Sharpie pen - Scissors - Pliers - Zip ties - 2 – 6V lantern batteries Shared Supplies - Power Strip - Hot Glue Gun - Glue Sticks - Drill - Drill bits (23/64, 5/16, 1/4, and one small) - Soldering Iron - Soldering Stand - Solder Rosin Core - Insulated Wire - Electrical Tape |
| --- | --- |
| # Instructor Prep | - Step 1 |
| # Related Resources | - List related lessons/ppts etc available from USC or outside, trusted orgs |

# Lesson Plan

## Introduction

Introduce students to the learning goals and make connections to past and present learning experiences. Stimulate interest and prompt students to identify their own questions about the topic. This can link the lesson to real world examples, introduce associated STEM careers, and assess their prior knowledge on the topic.

- Step 1

## Procedure

- Using ¼ drill bit, drill three holes in a straight line at the bottom of the plastic container about 1” apart.

- Drill three holes on one side of the plastic container about 1” apart (aligned to holes at bottom; horizontal alignment – 3 to 3 holes)

- Drill three holes on the other side of the plastic container about 1” apart (aligned to holes at bottom; horizontal alignment – 3 to 3 holes)

- Cut pieces of PVC to desired dimensions below and label each piece as follows:

| Length | Edge label |
| --- | --- |
| 8 ½” | Top left |
| 8 ½” | Top right |
| 6” | Front edge |
| 6” | Center edge |
| 5” | Bottom left |
| 5” | Bottom right |
| 4” | Front left |
| 4” | Front right |
| 4” | Back left |
| 4” | Back right |

- For both front left and front right PVC pieces, drill holes 1 ¼” from one end using the 5/16 drill bit; drill through the PVC.  Mark these places with an “M” (these positions will make sure that the propellers don’t make contact with the ROV). These holes are for the bolts to hold the motors.

- For the center edge PVC, drill a hole in center using the 5/16 drill bit; drill through the PVC.

- Label PVC elbows and tees as follows (using abbreviations)

| Elbow1 | Left front bottom (LFB) |  | Elbow8 | Right back bottom (RBB) |
| --- | --- | --- | --- | --- |
| Elbow2 | Right front bottom (RFB) |  | Elbow9 | Left center (LC) |
| Elbow3 | Left front top (LFT) |  | Elbow10 | Right center (RC) |
| Elbow4 | Right front top (RFT) |  | Tee1 | Left front (LF) |
| Elbow5 | Left back top (LBT) |  | Tee2 | Right front (RF) |
| Elbow6 | Right back top (RBT) |  | Tee3 | Left back (LB) |
| Elbow7 | Left back bottom (LBB) |  | Tee4 | Right back (RB) |

- Using 5/16 drill bit, drill holes in sides of and through LFB and RFB.

- Using 5/16 drill bit, drill holes in through curves of LFT, RFT, LBT, RBT (just top and not through PVC).

- Using 5/16 drill bit, drill holes in the center of the following:  top left, top right, bottom left and bottom right (without motors) for drainage and back left and back right.

- Using ¼ drill bit, drill holes through both sides of the film canister for bolts.  Repeat for a total of 3 film canisters.

- Using the pair of scissors, enlarge the hole on one side of the film canister (large enough to insert the head of the bolt through the hole).  Repeat for a total of 3 film canisters.

- Using the ¼ drill bit, drill a hole on the bottom of the film canister large enough for entire stem assembly to fit.  Repeat for a total of 3 film canisters.

- Using a small drill bit, drill to enlarge the hole of the propeller to fit the motor stem (drill through the hole of propeller).  Repeat for a total of 3 propellers.

- Using the scissors, make a hole in the center of the bottle cap for the wire to fit through (about ¼” hole).  Repeat for a total of 3 bottle caps.

ACTIVITY

TASK 1 – Assembling the Chassis of the ROV

- Insert front edge PVC into LFB and RFB.

- Insert 2” PVC connectors into LFB and RFB.

- With the LFB and RFB on their sides, connect LF to LFB and RF to RFB with the center of tees pointed up.

- Insert bottom left into other side of LF and bottom right into the other side of RF.

- Insert other side of bottom left into LB and the other side of bottom right into RB, with center of tees pointed up.

- Insert 2” PVC connectors into the other side of LB and RB.

- Connect LBB to the connector of LB with elbow pointed up.  Repeat for RBB to connector of RB.

- Insert 2” PVC connectors into top of tees LB and RB.

- With ends of elbows facing each other, connect LC to LB and RC to RB.

- Insert center edge into LC and RC.

- Insert back left into LBB and back right into RBB.

- Connect LBT to back left with LBT facing the front of ROV.

- Connect RBT to back right with RBT facing the front of ROV.

- Insert top left into LBT and top right into RBT.

- Connect LFT to top left and RFT to top right with drilled holes facing down.

- Insert front left into LFT and LF with drilled hole on the top.

- Insert front right into RFT and RF with drilled hole on the top.

TASK 2 – Assembling the Motors

- Use hot glue to cover all of the holes in the motors (total of 8 holes).  Be cautious to not get glue into the motor; just cover the holes with glue).

- Turn the stems to make sure glue did not get into the motors.  If the stem does not freely move, glue got into the motor and it must be replaced.

- Cut and strip 6 – 20 feet pieces of insulated wire.

- Solder wire to one of the wire holders of the motor.  Repeat for a total of 6 wires and 6 wire holders.

- Insert both sets of wire from the motor into the top of the bottle cap.  Repeat for a total of 3 bottle caps.

TASK 3 – Assembling Motors to the Chassis of the ROV

- Attach film canisters to front left, front right and center edge in marked places for motors by inserting bolt through side of film canister.

  - Attach nut and tighten with screw driver.

  - Make sure direction of motors allows movement up, left and right

- Insert motors into film canisters.  Make sure motor is straight; stem goes out of bottom and wires out of the open end.

- Insert bottle cap (with wires through them) into the film canister.

- Pull wires taut and glue bottle cap in place (use two pencils to hold bottle cap in place – one team member holds cap in place with pencils and another team member glues cap).

- Fill in exposed side hole of the film canister with glue.

- Repeat for a total of 3 film canisters.

- CAREFULLY squeeze a small drop of hot glue into propeller holes and attach to motor stems.

- Secure wires from motors to sides and bottom of ROV, away from propellers with zip ties

TASK 4 – Assembling the Remote Switches

- Strip 6 – 1’ red wires and 3 – 6” red wires.

- Strip 6 – 1’ black wires and 3 – 6” black wires.

- For each DPDT switch, solder 1 - 6” red wire to the center hole and 1 - 6” black wire to the other center (see diagram on next page).

- For each DPDT switch, solder a 1’ red wire to opposite corner holes of switch (see diagram on next page).

- For each DPDT switch, solder a 1’ black wire to opposite corner holes of switch (see diagram on next page).

Wiring for DPDT Switch

6” red wire

6” black wire

1’ red wire

1’ red wire

1’ black wire

1’ black wire

- Attach switches to bottom of plastic container by removing nuts from top of switches, inserting into hole at bottom of containers and reattaching bolts.

- For each switch pull both 6” wires through corresponding hole in back.

- For each switch, pull all four 1’ wires through corresponding holes in front.

- Attach red 6” wire from center switch to (+) terminal of 6V lantern battery and black 6” wire from center switch to (-) terminal of battery.

- Twist together red 6” wires from outer switches and attach to (+) terminal of second 6V lantern battery; and twist together black 6” wires from outside switches and attach to (-) terminal of second battery.

- Secure batteries and wires to container by using glue and electrical tape.

- For each switch, twist together 2-1’ red wires.

- For each switch, twist together 2-1’ black wires.

- Twist red 1’ wires from center switch to one of the 20’ wires of up/down motor.

- Twist black 1’ wire from center switch to remaining 20’ wire of up/down motor.

- Twist red 1’ wires from left switch to one of the 20’ wires of left motor.

- Twist black 1’ wire from left switch to remaining 20’ wire of left motor.

- Twist red 1’ wires from right switch to one of the 20’ wires of right motor.

- Twist black 1’ wire from right switch to remaining 20’ wire of right motor.

- Insert batteries into battery holders.

- Test movement of propellers; make sure propellers move in both directions.

- Use electrical tape to secure red and black wires to 20’ wires and to secure battery holder wires to container.

- Immerse ROV into water and adjust buoyancy so ROV floats about 6” below surface by:

  - Adding pipe insulator (cut to size and attach with zip ties).

  - Adding washers with bolts to front elbows or back vertical edge.

## Wrap-up

End the lesson with some final discussions about what students designed/discovered. Connect their activity back to the focus question/problem, real world examples, and STEM careers.

Also include notes about any special cleanup considerations

# Lesson Background for Teachers

## Suggested Real-World STEM Connections

- List possible related Role Models in STEM (with details and links to more reading). Give suggestions for examples of scientists/engineers who are from underrepresented groups in STEM in related careers.

- List related real-world engineering problems and examples

## Lesson Variations and Options

Include descriptions of alternative methods or ways to present things.

## Explanation

Describe the background concepts and explanations behind the lesson.  This can be more in depth than a teacher/instructor would actually share with students.

## Key Concepts and Vocabulary

- Word 1: Grade Appropriate Definition

- Word 2: Grade Appropriate Definition

## Safety Notes

- Safety instruction