# Balloon Jet

| Subject: Aerospace Engineering Related Subjects: Physics, Aerodynamics | Grade Level(s): 4-7 Length of Lesson: 45 minutes | Type: Project Keywords: Thrust, Newton’s Laws, Air Pressure, Flight, Forces |
| --- | --- | --- |

# Lesson Overview

In this project, students will build a simple balloon-powered glider using cut-out parts, a straw, and a balloon. This activity introduces students to basic principles of propulsion, motion, and aerodynamics through a fun, hands-on experiment.

# Lesson Focus

How can we use air pressure and basic flight components to create a balloon-powered flying jet?

| # Lesson Objective(s) | - Build a balloon-powered jet using simple materials. - Understand how thrust, drag, and lift affect motion. - Explore Newton’s Third Law in a real-world application. |
| --- | --- |

# Lesson Timing

| 5 minutes | Introduction to propulsion and flight |
| --- | --- |
| 10 minutes | Material distribution and explanation |
| 20 minutes | Construction of the balloon jet |
| 10 minutes | Launching, testing, and reflection |

| # Materials | - Rolls of masking tape - Balloon Jet cut-out - Balloons - Straws - Scissors |
| --- | --- |
| # Instructor Prep | - Print and cut Balloon Jet templates (side 1 and side 2) ahead of time. - Set up a launch lane or clear space in the classroom or hallway. - Demonstrate an assembled model to explain the placement of each component. |
| # Related Resources |  |

# Lesson Plan

## Introduction

Ask students:

- How do rockets and jets move forward?

- What happens when you release air from a balloon?

Introduce Newton’s Third Law: “For every action, there is an equal and opposite reaction.” Explain how inflating and releasing the balloon causes air to rush out and propel the jet in the opposite direction.

## Procedure

- Cut out glider pieces for side one and two

- Tape the pieces of side one with their corresponding pieces of side two

- Tape the pieces onto the straw:

  - Tape the wings 6cm from the end of the straw(the larger side should be taped on the straw)

  - Tape the horizontal stabilizers 4 cm from the end of the straw(on the opposite end from the wings)

  - Tape the vertical stabilizers vertically on the straw between the horizontal stabilizers

- Tape the balloon on the end of the straw closest to the wings

- Blow on the opposite end of the straw and inflate the balloon and release the glider

## Wrap-up

Ask students:

- How did the balloon make the jet move?

- What part of the jet helped keep it stable?

- How would moving the wings or fins affect the flight?

Discuss the forces involved (thrust, drag, lift, gravity) and how this applies to real aircraft and rockets. Encourage redesigns and retests.

# Lesson Background for Teachers

This activity is a simple demonstration of thrust and flight control. When air escapes the balloon, it pushes the glider in the opposite direction. The design of the wings and stabilizers helps maintain balance and direction. This replicates basic physics and aerospace principles.

## Lesson Variations and Options

- Let students experiment with different wing placements.

- Use longer straws or larger balloons and compare flight distances.

- Create a balloon jet race or accuracy challenge.

## Explanation

The balloon stores potential energy when inflated. When released, the air escapes and produces thrust, pushing the jet forward. Wings and stabilizers manage lift and stability, helping the glider fly smoothly.

## Key Concepts and Vocabulary

- Thrust: The force that propels an object forward

- Lift: The upward force that helps an object stay in the air

- Drag: Air resistance against motion

- Stability: The ability to maintain controlled flight

## Safety Notes

- Ensure students do not blow too forcefully or inhale through the straw.