# Cotton Candy

| Subject: Chemistry, Physics, Chemical Engineering, Electronics Related Subjects: Changing states of matter, centrifugal force, electricity, temperature and heat | Grade Level(s): Length of Lesson: | Type: Inquiry / Design / Project Keywords:Matter, solid, liquid, gas, centrifuge, series circuit, |
| --- | --- | --- |

# Lesson Overview

Students create a cotton candy machine from a soda can, wooden dowel, 1.5V motor and lots of sugar. Students then will be able to make cotton candy at will (much to their parents’ chagrin).

# Lesson Focus

What is the central question or phenomenon being investigated?  What problem are they trying to solve?

| # Lesson Objective(s) | - Students create a series circuit with a motor, battery holder and moment switch. - Students create candy making unit by connecting soda, can dowel, series circuit. - Students demonstrate change of state of matter by melting sugar. - Students demonstrate centrifugal force by spinning can. - Students create cotton candy. - Student constructs fire safe box and extinguisher. |
| --- | --- |

# Lesson Timing

| 5 minutes | Introduce the Lesson |
| --- | --- |
| 30 minutes | ... |

| # Materials | - –Soda can 17. - pick - –wood dowel 18. –coffee can - –1.5v motor 19. - alcohol - – battery pack 20. - sugar - –push button switches 21. - paper - –wire 22. – glue gun - –small nails 23. – glue sticks - –pliers 24. – small pieces of wood - –wire cutter/stripper - –hammer - –push pins - –box - –soldering iron - –solder - –batteries - –aluminum foil |
| --- | --- |
| # Instructor Prep | - Step 1 |
| # Related Resources | - List related lessons/ppts etc available from USC or outside, trusted orgs |

# Lesson Plan

## Introduction

# For years, cotton candy has been one of the mainstays at carnivals, circuses and fairs across the country.  This delectable treat has served to delight kids of all ages.

## Procedure

- –Cut 2 pieces of wire 8 inches each.

- – Strip ½ inch on each end of each wire.

- –Using the soldering iron solder wires to motor.

- –Cut 12” wood dowel.

- –Using push pin, make a hole in both ends of dowel.

- –Using push pin, make a hole in the bottom of the soda can.

- –Using wire cutters, clip the center of soda can tab so dowel can slide through.

- –Slide dowel through tab and align hole in dowel with hole in can.

- –Turn can upside down (with dowel inside), insert small nail through hole in can into dowel and hammer in place.

- –Enlarge hole on other in of dowel with pick.

- –Glue motor stem into hole in dowel and glue soda can tab to side of hole at the top of the can.

- – Make small holes on the bottom sides of the can with the push pin.

- –Insert push pin, at the top of the dowel, where the stem is inserted , to  secure motor.

- –Connect one wire from motor to one wire from battery pack.

- –Solder two wires to the push button switch (if necessary).

- –Connect remaining wire from motor to wire from push button switch.

- –Connect remaining wires from push button switch and battery pack together.

- –Glue battery pack and switch to small piece of wood.

- –Cut second soda can in half.

- –Cut Coffee can slightly higher than half soda can.

- – Tape aluminum foil to the inside of the box, ensuring that only the aluminum foil is exposed. Cover the entire inside of the box.

- –Secure coffee can to bottom of the box with tape and aluminum foil.

- –Place half soda can inside coffee can and fill ¾ way with alcohol

- –Cut wood large enough to cover mouth of coffee can and glue dowel for handle (This is your fire extinguisher).

- –Insert batteries into battery pack.

- –Pour sugar into can

- –Ignite alcohol with match.

- –Hold can above flame and press push button switch.

- –If the can spins too fast, hold button for 3 seconds and release it for 2 seconds.

- –The resulting web like substance is cotton candy.

- –Extinguish flame.

- –Collect cotton candy

## Wrap-up

- Students should be able to explain the three states of matter and the cause of a change of state.

- Students should be able to cite examples of centrifugal force.

- Students should be able to give other examples of applications of chemical engineering machines.

# Lesson Background for Teachers

## Suggested Real-World STEM Connections

- List possible related Role Models in STEM (with details and links to more reading). Give suggestions for examples of scientists/engineers who are from underrepresented groups in STEM in related careers.

- List related real-world engineering problems and examples

## Lesson Variations and Options

Include descriptions of alternative methods or ways to present things.

## Explanation

Most substances exist in three states: solid, liquid and gas.  Most also have a melting and a freezing (solidifying) point.   Increasing or decreasing temperature causes most substances to go from one state to another.

To make cotton candy, first sugar is placed in a can and held over a heat source.  The sugar melts and becomes liquid.  At the same time, this sugar is rapidly spun around inside the can.  Centrifugal force pushes the liquid sugar to the wall of the can.  Since the can has small holes, around the sides, near the bottom (previously prepared by student), the liquid sugar is propelled out of the can at a high rate of speed.  The liquid sugar cools as it gets further from the heat source and solidifies (becomes solid) creating streams of solid sugar that look like spider webs.  This is the cotton candy!

There are three elements that make popcorn work like this:

* Moisture inside the kernel

* Starch inside the kernel

* The hard shell surrounding the kernel

Example:

Water (H2O)

- Freezing/Melting Point (solid to liquid) = 0° C / 32° F

Sugar (Sucrose, commonly known as table sugar – C12H22O11)

- Melting Point (solid to liquid) = 160° - 186° C / 320° - 367° F

## Key Concepts and Vocabulary

- Word 1: Grade Appropriate Definition

- Word 2: Grade Appropriate Definition

## Safety Notes

- EXTRA CAUTION must be observed by students while working with alcohol and fire.  Adult supervision is strongly recommended.

- Students must be careful with hot glue guns.

- Students must be careful with soldering iron.

- Students must be careful with all sharp objects and instruments.