# Electronic Plinko

| Subject: Physics, Mathematics Related Subjects: Gravity, series circuit, probability, collisions | Grade Level(s): Length of Lesson: | Type: Inquiry / Design / Project Keywords: Diode, randomness, contact point, elastic collision |
| --- | --- | --- |

# Lesson Overview

Students will create a pattern of nails on an inclined board.  The pattern will allow multiple paths for a marble to roll. There are three locations where if the marble travels, an LED will turn on.

# Lesson Focus

What is the central question or phenomenon being investigated?  What problem are they trying to solve?

| # Lesson Objective(s) | - Students will create a game using gravity as the force used to move a marble through a maze of nails. - Students will create 3 series circuits that will be triggered by the marble. |
| --- | --- |

# Lesson Timing

| 5 minutes | Introduce the Lesson |
| --- | --- |
| 30 minutes | ... |

| # Materials | - –Red LED - –Yellow LED - –Green LED - –100 5/8” nails - –6 7/8” nails - –Aluminum foil - –Battery holder - –Drill - –3/16” drill bit - –Hot Glue Gun - –Glue sticks - –Insulated wire - –Foam board or cardboard - –Rubber bands - –Plywood - –Saw - –Hammer - –Soda can - –Wire cutters/stippers - –Soldering Iron - –Solder - –Ruler - –Pencil |
| --- | --- |
| # Instructor Prep | - Step 1 |
| # Related Resources | - List related lessons/ppts etc available from USC or outside, trusted orgs |

# Lesson Plan

## Introduction

# Patterned after a game on a popular TV game show, Electronic Plinko is a fun way for kids to learn about circuits, diodes, gravity, and probability

## Procedure

- –Cut 12” x 10” (L x W) board.

- –Drill a hole 4 inches from top (length wise) in the center (width wise) of board.

- –Drill a hole 2 inches from center hole on each side.

- –Two inches below holes, mark off a ½” square grid as shown in figure1 making 10 rows.

- –Hammer 5/8” nails in the corners as shown in figure 2 for the first 3 rows (don’t hammer nails through board).

- –For the middle next two rows randomly place nails (leave enough room for marble to roll through.

- – Hammer nails in the corners as shown in figure 2 for the next 3 rows.

- –For the last two rows randomly place nails (leave enough room for marble to roll through (see figure 3).

- –Remove 3 sets of 2 diagonal nails and replace with 7/8” nails.

- –Hammer 7/8” nails through plywood with ¼” sticking out back.

- –Glue 1” foam board (or cardboard) stop at bottom of plywood.

- –Glue Soda can to back of plywood at the top (creating an incline)

- –Wrap rubber band along nails on outside of each side of plywood (creating a side stop).

- –Insert top of an LED into each hole.

- –Position each LED so the long lead is on the same side for all.

- –Glue LED in place (you only need a drop or two of hot glue).

- –Cut three strips of aluminum foil 3/8” wide and  long enough to reach from one nail pair to other.

- –Wrap foil around one nail in each pair.  Foil should be like a swinging saloon door. From one nail to other of pair.

- –Cut 4 – 8” wires and strip both ends.

- –Cut 5 – 5” wires and strip both ends.

- –Glue battery holder to top, front of board.

- –Wire connections are all on back of board.

- –Using an 8” wire, connect the long lead of the Red LED to one of the foil nails.

- –Do the same for the Yellow and Green LEDs.

- –Connect the non-foil nails to each other using 2 – 5” wires.

- –Connect one of the non-foil nails to the positive wire of the battery holder using the last 8” wire.

- –Connect the short leads of the LEDs together using 2 – 5” wires.

- –Using the last 5” wire, connect one of the short leads to the negative battery holder (see figure 4).

- –Solder wire connections.

- –Insert batteries and roll marble.

Figure 4

Figure 3

## Wrap-up

- Students should be able to explain series circuit.

- Students should be able to explain probability.

- Students should be able to explain why marble goes in different directions.

# Lesson Background for Teachers

## Suggested Real-World STEM Connections

- List possible related Role Models in STEM (with details and links to more reading). Give suggestions for examples of scientists/engineers who are from underrepresented groups in STEM in related careers.

- List related real-world engineering problems and examples

## Lesson Variations and Options

Include descriptions of alternative methods or ways to present things.

## Explanation

On a fixed area inclined board with a specific number of nails, there are a fixed number of paths that a marble can travel, given a specific starting point.  By simply rolling a marble on the board, the path it takes seems to be random.  However, its path is due to a number of elastic collision (collisions without deforming the object) that causes the direction of force, caused by gravity, to change.  There is an equal chance that the marble takes a particular path.  Using this phenomenon, we can create a fun game of chance.  We give varying points for the marble taking a particular path.  We show these paths by making an LED light up if the marble goes that way.

An LED (light emitting diode) is a type of circuit that gives off light.  In order for it to light up, one end of the LED (usually the long end) must always be connected to the positive end of a battery.  With most light bulbs, it doesn’t matter which way you connect them.

Students cut a 12” x 10” piece of plywood.  Students then hammer 1/8” of 5/8” nails into 10 rows following a regular pattern for 7 rows and random placement for 3 rows.  Six of the nails will be replaced by 7/8” nails later.  A popular pattern is to make ½” squares and place the nails in the corners (See figure 1).

3 Pairs of diagonal nails are then replaced by 7/8” nails, which will be hammered through the board (see figure 2). A strip of aluminum foils is then wrapped around one nail of each pair.  The foil must be wrapped in so that it can swing to create a contact point with the second nail of the pair.

Figure 2

Figure 1

# The nail with the foil will be the positive contact for the LED.  The positive lead of the red LED will be wired to one of the “foil nails”.  The same will be done for the green and yellow LEDs. The other lead of the LEDs will be wired together and connected to the negative lead of the battery holder.  The positive lead of the battery holder will be wired to all three “non-foil” nails of the 3 pairs of 7/8” nails. When the marble rolls between the two nails of the pair, it will force the foil to make a contact between the two nails, thus closing the circuit for the LED and making it light up.

## Key Concepts and Vocabulary

- Word 1: Grade Appropriate Definition

- Word 2: Grade Appropriate Definition

## Safety Notes

- Students should tie back hair and loose clothing.

- The glue gun and soldering iron extremely hot.  Students should wear goggle when using both and exercise extreme caution.

- Students should exercise caution when using hammer and nails.

- Students should use caution when drilling.