# Hand Fan

| Subject: Physics, Mechanical Engineering Related Subjects: Electrical Engineering | Grade Level(s): 6-10 Length of Lesson: 75 minutes | Type: Project Keywords: Electricity, Circuits, Motor, PVC Construction |
| --- | --- | --- |

# Lesson Overview

Students will learn how to build a working electric fan using basic electrical components and PVC pipe. This activity introduces students to concepts in circuits, energy transfer, and mechanical motion through hands-on construction.

# Lesson Focus

How do motors, wires, switches, and batteries work together to power a simple machine? What is needed to complete a functional electrical circuit?

| # Lesson Objective(s) | - Construct a working electric fan using a motor and PVC structure. - Understand how current flows through a closed circuit. - Learn to connect and troubleshoot electrical components safely. - Explore how mechanical and electrical systems interact. |
| --- | --- |

# Lesson Timing

| 10 minutes | Introduction and safety briefing |
| --- | --- |
| 50 minutes | Step-by-step fan construction |
| 15 minutes | Testing, adjustments, and discussion |

| # Materials | Per Student or Team - Motor - 1, long 8 inch pvc - 1 pvc tee - 1, pvc elbow - 1, 1-AA battery holder - 1, AA battery - 1, push button - 1, propeller - Ruler - Drill with 5/16 drill bit - A boring bit - Wire cutter - Black and Red wire - Electric tape Shared Tools - Drill with 5/16” drill bit - Boring bit - Wire cutters/strippers - Ruler |
| --- | --- |
| # Instructor Prep | - Pre-cut wires and prepare materials per kit. - Set up drilling stations with supervision. - Demonstrate battery polarity and motor wiring before build starts. |
| # Related Resources |  |

# Lesson Plan

## Introduction

Discuss what a motor is and how electricity powers simple machines. Introduce the circuit path: battery → switch → motor → battery. Show a complete fan model and explain how the parts work together.

## Procedure

- Start with drilling a hole into the 8 inch pvc pipe 5 inches from the top and 3 inches from the bottom. Start the hole using the boring bit to guide the drill bit.

- Cut wires: one 12 inch black wire, one 10 inch red wire, and one 5 inch red wire. Use a wire cutter to strip the ends of both ends of the wire.

- Connect the two red wires to the posts of the push button, one on each side and tape them both individually.

- Pull the wires of the push button through the hole in the pvc pipe

- Pull the 12 inch wire of the push button through the opening on the longer end of the pipe (from the hole) and the 10 inch wire through the other end.

- Insert the push button into the hole.

- Put the battery holder with the battery already inside through the base of the pvc tee (if the pvc tee opening is too small, you will need to use the battery without the battery holder).

- Connect the 12 inch black wire to the black wire of the battery holder by twisting them together and tape them.

- Pull the black 12 inch wire connected to the black wire of the battery holder through the top of the pvc tee and through the pvc pipe.

- Connect the 3 inch red wire of the push button to the red wire of the battery holder and tape them.

- Attach the pvc pipe to the pvc tee.

- Pull both the red wire and the black wire through the pvc elbow

- Attach the pvc pipe to the pvc elbow

- Connect the black and red wires separately to each post of the motor.

- Attach the propeller to the motor.

- Press the button and see if the air blows to you. If it doesn’t switch the red and black wires

- Tape the wires

- Add tape around the motor to make it snuggly to fit into the pvc elbow.

- The fan will begin to spin when the button is pushed.

## Wrap-up

Have students explain how electricity flows through their fan circuit. Ask what part of the build was most challenging and how they overcame it. Discuss real-world applications of motors in daily life.

# Lesson Background for Teachers

## Suggested Real-World STEM Connections

- Home Appliances: Many household items (fans, blenders, hairdryers) use simple electric motors like the one in this project, demonstrating everyday applications of small-scale motor circuits.

- HVAC Systems: Heating, ventilation, and air conditioning units rely on electric fans to circulate air—similar to the fan students build, but scaled up with advanced control systems.

- Renewable Energy Systems: Wind turbines convert mechanical energy from rotating blades into electricity—the reverse of what students build, offering a great comparison to motor-generators.

- NASA and Aerospace Cooling Systems: Spacecraft require small, efficient fans and motors to regulate temperature in low-gravity environments, showcasing the importance of compact motorized systems.

STEM Careers:

- Electrical Engineers – design and optimize circuits and devices powered by electricity.

- Mechanical Engineers – work on systems that use motors to create motion or airflow.

- Industrial Designers – design functional, safe, and efficient small devices like fans and hand tools.

## Lesson Variations and Options

- Replace the push button with a toggle switch for continuous use.

- Add decorative fan blades or housing for aesthetic design challenges.

- Use solar panels in place of batteries to discuss renewable energy.

## Explanation

The fan is powered by a closed circuit formed by a battery, switch, wires, and motor. Pressing the push button completes the circuit, allowing electrical current to flow and power the motor. The rotational energy produced by the motor turns the propeller, pushing air and generating a breeze. Reversing wire polarity changes the motor’s spin direction, showing current flow’s impact on motor function.

## Key Concepts and Vocabulary

- Circuit: A complete path for electrical flow

- Motor: Converts electrical energy into mechanical motion

- Switch: Opens/closes a circuit to control flow

- Polarity: Direction of current flow

- Conductivity: Material’s ability to carry current

## Safety Notes

- Supervise use of drills and cutting tools.

- Ensure all connections are taped to prevent shorts.