# Motor - I’ve got the power!

| Subject: Physics Related Subjects: Magnetism, electromagnetism and momentum | Grade Level(s): 1,2,3,4,5,6,8 Length of Lesson: | Type: Inquiry / Design / Project Keywords:Magnet, pole, polarity, electromagnet, motion |
| --- | --- | --- |

# Lesson Overview

# Electric motors are a large part of our lives; from refrigerators, to electric cars, to fans, to elevators.   To make them work, we rely on the attraction and repulsion of magnets and the uses of electromagnets.

# Permanent magnets have two poles, North and South (or charges, + and -).  The north pole of a magnet is attracted to the south pole and repels the north pole of another magnet.  The south pole of a magnet is attracted to the north pole and repels the south pole of another magnet.

# Ampere’s circuital law shows that current flowing in a coil of wire makes the coil of wire a magnet (an electromagnet to be exact).  The more coils the wire has, the stronger the magnet.

# When a battery is connected to the coil of wire, an electromagnet is formed and is either attracted to or repelled from the permanent magnet. This attraction or repulsion causes the coil to move.  We take advantage of this by positioning the coil so that it can spin freely above the permanent magnet and its momentum keeps it going.

# Lesson Focus

Students will create a motor from a paper cup, wire, and a magnet.  It will demonstrate how an electric motor works.

| # Lesson Objective(s) | - Students will observe the creation of an electromagnet. - Students will create paper cup motor. |
| --- | --- |

# Lesson Timing

| 5 minutes | Introduce the Lesson |
| --- | --- |
| 30 minutes | ... |

| # Materials | - Paper cups - Masking tape - Magnet wire - Permanent magnets (1”x.75” rectangle) - Sandpaper - Wire cutters - Solder - Soldering Iron - Batteries - Aluminum foil - Insulated wire - Paper clips - Film canisters - Cardboard - Glue gun and glue sticks |
| --- | --- |
| # Instructor Prep | - Step 1 |
| # Related Resources | - List related lessons/ppts etc available from USC or outside, trusted orgs |

# Lesson Plan

## Introduction

An electrical current travels through the coil of wire creating an electromagnet. The coil of wire moves because of the attraction or repulsion (depending on how battery is connected) of it as an electromagnet to the permanent magnet.  Because the coil is mounted above the permanent magnet and allowed to freely rotate in its conducting cradle (see figure 1), its like pole is repulsed and its  opposite pole is attracted to the permanent magnet.  The coil then rotates to reorients itself.  However, the momentum the rotation combined with the force of motion causes the coil to lose contact with the cradle for a split second returning it to a non magnetic coil.  Since it is still moving,  the coil continues to rotate, falls back into place, returns to its original position and re-contacts the cradle.  This causes the rotation to begin again.

## Procedure

- –Cut 2 pieces of 2ft of magnet wire.

- –Sand about ½” of ceramic coating from each end on both wires.

- –Using one of the wires, make one big loop by twisting the ends of the wire (see Figure 2)

- –Place a permanent magnet on the table.

- –Hold the loop over the magnet about ½”.

- –Connect battery and watch movement of loop toward or away from permanent magnet.  This is a weak electromagnet.

- –Try to pick up paper clip with loop electromagnet.  It can’t!

- –Wrap the other wire around a nail.

- –Connect battery and try to pick up paper clip.  It works!

- –Try to pick up paper clip with another plain nail. Nothing!

- –The only difference between the two electromagnets is the number of loops.  Hence, the more loops the stronger the electromagnet.

- –Repeat steps 6 through 11 with stronger batteries.  Therefore, more current, stronger electromagnet

- –Cut a 2ft piece of magnet wire

- –Sand about ½” of ceramic coating from each end.

- – Leaving about 1 inch from the start and on the end, wrap the magnet wire around  (see Figure 2)

- –Remove coil from film canister and wrap ends around coil once to hold its shape.

- –Unbend bottom of two paper clips.

- –Bend about 1cm of each in a right angle (see figure 3)

- –Tape paper clips on cup, across from each other, with bent end laying flat on surface of cup.  They should extend upward from the bottom of the cup (see figure 4).

- – Drop one permanent magnet inside cup.

- –Place another permanent magnet on bottom of cup and hold until both magnets attract.

- –Mount coil in paper clip cradle as shown in figure 1.

- –Coil should be able to spin freely.

- –Cut 2, 10” pieces of insulated wire.

- –Strip both ends of both wires.

- –Attach small pieces of aluminum foil to one end of each piece of wire.

- –Attach other ends of wires to each paper clip cradle piece.

- –Mount cup and wires to cardboard object with glue and tape.

- –Squeeze aluminum foil ends on battery and watch spin.

- –You may need to adjust height or location of paper clip cradle and/or  shape or position of coil to get spinning effect.

- –To get it started, you may need to give it a gentle tap.

## Wrap-up

- Students should be able to explain why spinning occurs.

- Students should be able to troubleshoot and correct problems.

- Students should change variables and observer results

- Size or charge of battery

- Number of magnets

- Number of coils of wire

# Lesson Background for Teachers

# Safety Procedures

- Students should exercise caution when using sandpaper and wire cutters

- Students must be careful when using soldering iron