# Newton’s Cradle

| Subject: Physics, Mathematics Related Subjects: Civil Engineering | Grade Level(s): 5-8 Length of Lesson: 90 minutes | Type: Project Keywords: Newton’s Laws, Momentum, Energy Transfer, Conservation |
| --- | --- | --- |

# Lesson Overview

In this project, students will build a simple Newton’s Cradle to explore the concepts of momentum, conservation of energy, and Newton’s Third Law. The activity will help visualize how energy is transferred between objects through collisions.

# Lesson Focus

How is energy transferred between objects in motion? What does Newton’s Cradle demonstrate about momentum and forces?

| # Lesson Objective(s) | - Build a working Newton’s Cradle using common materials. - Understand energy conservation and momentum transfer in a closed system. - Explore Newton’s Third Law through hands-on experimentation. |
| --- | --- |

# Lesson Timing

| 5 minutes | Introduce the Lesson |
| --- | --- |
| 15 minutes | Material distribution & safety briefing |
| 40 minutes | Build Activity |
| 15 minutes | Testing, adjusting, discussion, and wrap-up |

| # Materials | - Pushpins (#20) - Large paper clips(#20) - Marbles(#20) - (1”x1”x36”) Balsawood (#3) - (⅛”x36”) Balsawood (#20) - Hot Glue Guns (#2) - Hot Glue Gun sticks (#4) - Ruler (#10) - Straws (#20) |
| --- | --- |
| # Instructor Prep | - Pre-cut balsawood if time is limited or prepare measuring instructions. - Set up glue stations for safety and efficiency. - Print or project diagrams for frame assembly. |
| # Related Resources |  |

# Lesson Plan

## Introduction

Introduce students to Newton’s Cradle and the concept of energy transfer through momentum. Ask:

- What happens when one marble hits the rest?

- How does energy move in a collision?

- Use videos or real Newton’s Cradle demos to engage curiosity.

## Procedure

- Cut one 4” piece of 1”x1” Balsawood

- Cut four 12” pieces of ⅛” Balsawood

- Cut three 5” pieces of ⅛” Balsawood

- On 4” Balsawood, draw a line lengthwise ¼ “  from an edge (the edge with the line closest will be the top and side with the line will be the front of)

- Insert 4 push pins ½”; apart and remove

- Insert large paper clips into one end of each straw

- Glue marbles on other end of each straw

- Glue each 12”  piece (the legs) of Balsawood to the four corners of the bottom of the 4”; piece.

- Using the 5”; Balsa Wood pieces, create a capital “I”

- Glue the capital I to the 4 legs to make the cradle stable

- Insert the metal part of the thumbtack through the paperclip attached to one of the straws and then

reinsert both into previously made hole

- Repeat for other straws

- Adjust so that the marbles can swing freely and hit each other

## Wrap-up

- Observe the energy transfer through the cradle.

- Discuss how it reflects Newton’s Third Law.

- Ask: What would happen if the middle marble were removed? What if the cradle used heavier/lighter marbles?

- Connect to real-world applications like bumper design, sports collisions, and physics of motion.

# Lesson Background for Teachers

Newton’s Cradle demonstrates the conservation of momentum and energy through elastic collisions. It’s a practical illustration of Newton’s Third Law: “For every action, there is an equal and opposite reaction.” This activity makes abstract physics concepts tactile and intuitive.

## Suggested Real-World STEM Connections

- Role Models:

  - Mae Jemison (Engineer & Astronaut)

  - Dr. Arlyne Simon (Biomedical engineer and inventor)

- Applications:

  - Car crash safety systems

  - Billiards/pool physics

  - Robotics and automation dynamics

## Lesson Variations and Options

- Use ping pong balls and string to simulate similar effects with lighter materials.

- Turn it into a group challenge: build the most efficient cradle using a limited material budget.

- Incorporate math by calculating potential energy and measuring swing time.

## Explanation

This lesson explores conservation laws in physics: energy and momentum are conserved in ideal systems. When one marble strikes the others, its energy is transmitted through the chain, propelling the one on the opposite end. Friction, air resistance, and imperfect elasticity reduce efficiency in real-world models.

## Key Concepts and Vocabulary

- Momentum: The quantity of motion of a moving body

- Conservation of Energy: Energy cannot be created or destroyed

- Elastic Collision: A collision where there is no loss of kinetic energy

- Newton’s Third Law: Every action has an equal and opposite reaction

## Safety Notes

- Do not swing or throw marbles outside the designed setup.

- Dispose of broken materials properly.