# Penny Battery

| Subject: Related Subjects: | Grade Level(s): Length of Lesson: | Type: Inquiry / Design / Project Keywords: |
| --- | --- | --- |

# Lesson Overview

Batteries are an important piece to all portable electronic devices. They store and supply electrical energy when an appliance cannot use AC power outlets. They convert chemical energy into electrical energy. Batteries are made of cells, each of which have an anode (positive side), and a cathode (negative side) and they are stacked in series. A “salt bridge” between the two terminals is necessary to allow the flow of charged ions and thus, an electrical potential difference (voltage).

# Lesson Focus

What is the central question or phenomenon being investigated?  What problem are they trying to solve?

| # Lesson Objective(s) | This project should help students understand how batteries work. What is important is how the batteries convert chemical potential into electrical potential, how they have two distinct terminals, and how the salt bridge is necessary. |
| --- | --- |

# Lesson Timing

| 5 minutes | Introduce the Lesson |
| --- | --- |
| 30 minutes | ... |

| # Materials | - Pennies - Zinc Washers - Paper Towel/Cardboard - Vinegar - Scissors - Aluminum foil (for the terminals) - Electrical Tape - Voltmeter and/or LED |
| --- | --- |
| # Instructor Prep | - Step 1 |
| # Related Resources | - List related lessons/ppts etc available from USC or outside, trusted orgs |

# Lesson Plan

## Introduction

Introduce students to the learning goals and make connections to past and present learning experiences. Stimulate interest and prompt students to identify their own questions about the topic. This can link the lesson to real world examples, introduce associated STEM careers, and assess their prior knowledge on the topic.

- Step 1

## Procedure

- Begin soaking small pieces of cardboard/paper towel in vinegar (about the size of the zinc washer).

- Students will start with a piece of electrical tape and a thin piece of aluminum foil that together form a ‘L’ shape (aluminum foil being the shorter side).

- Where the foil and electrical tape meet, they will begin stacking their cells. The cells must go in order zinc washer -- vinegar soaked cardboard/paper towel – penny and repeat.

- Stack three cells, then place another thin piece of aluminum foil on the top of the stack.

- Tape around the whole stack so that it is all covered in electrical tape with the exception of the two aluminum foil strips sticking out from either end. These will act as the two terminals. Make sure the aluminum foil strips are only touching either the zinc or penny on its respective end; they cannot be touching other components inside the battery or else the battery will not be effective.

- Measure the voltage between the two terminals! If possible, light up an LED. Students can put their batteries that they made in series with each other’s to get enough voltage to light up the LED.

## Wrap-up

End the lesson with some final discussions about what students designed/discovered. Connect their activity back to the focus question/problem, real world examples, and STEM careers.

Also include notes about any special cleanup considerations

# Lesson Background for Teachers

## Suggested Real-World STEM Connections

- List possible related Role Models in STEM (with details and links to more reading). Give suggestions for examples of scientists/engineers who are from underrepresented groups in STEM in related careers.

- List related real-world engineering problems and examples

## Lesson Variations and Options

Include descriptions of alternative methods or ways to present things.

## Explanation

Describe the background concepts and explanations behind the lesson.  This can be more in depth than a teacher/instructor would actually share with students.

## Key Concepts and Vocabulary

- Word 1: Grade Appropriate Definition

- Word 2: Grade Appropriate Definition

## Safety Notes

- Safety instruction