# Rover

| Subject: Mechanical advantage, motion, electricity, friction, torque Related Subjects:Physics, Mechanical Engineering, Electronics | Grade Level(s): Length of Lesson: | Type: Inquiry / Design / Project Keywords: |
| --- | --- | --- |

# Lesson Overview

Anyone who has been on a bike or in a stick shift car has felt the effects of gearing and

mechanical advantage causing motion. Some energy source causes an object to turn. That object

turns another object causing the bike or vehicle to move forward.

# Lesson Focus

Students will construct a chassis, wheels, gearing system and an electrical circuit in a car that

actually works. Students will test cars for speed and stability.

| # Lesson Objective(s) | • Students build suspension (wheels and axles) from cans or bottles, foam core and wood dowels. • Students build chassis from wood base shoe and foam core. • Students create series circuit with motor, batter holder and switch • Students assemble USC rover. • Students demonstrate mechanical advantage by connecting gears (motor stem to can wheel) via rubber band. • Students determine which design leads to better performance in speed, direction, incline, and “off road” conditions • Students will also demonstrate trade-off of style versus function |
| --- | --- |

# Lesson Timing

| 5 minutes | Introduce the Lesson |
| --- | --- |
| 30 minutes | ... |

| # Materials | • Foam Board • Wood sticks or PVC pipe • Wood Dowels • Saw • Wire cutters • Wire • Wire stripper • Pencils • Motors • Battery holders • Batteries • Rulers • Xacto knife • Switches |
| --- | --- |
| # Instructor Prep | - Step 1 |
| # Related Resources | - List related lessons/ppts etc available from USC or outside, trusted orgs |

# Lesson Plan

## Introduction

Introduce students to the learning goals and make connections to past and present learning experiences. Stimulate interest and prompt students to identify their own questions about the topic. This can link the lesson to real world examples, introduce associated STEM careers, and assess their prior knowledge on the topic.

- Step 1

## Procedure

1. Cut four 2” squares of foam board

2. Glue squares to both ends of each soda can

3. Cut 2 dowels about 3 inches longer than the soda cans.

4. Drill a hole in both ends of both soda cans (through the foam core).

5. Insert a dowel through both holes of one soda can (this is the wheel- axle)

6. Repeat step for the other can.

7. Cut 1-6”x 2” foam board rectangle

8. Cut a “V” in the center of one side of rectangle

9. Cut 2 pieces of wood sticks or pvc. The wood must be long enough for both cans to spin

freely when connected to the chasis.

10. Drill holes for axle in both sides of both pieces of wood or pvc

11. Glue the foam core to the pieces of wood using the “H” shaped model.

12. Strip the wires of a battery holder.

13. Cut and strip a 4” wire

14. Twist one wire to one lead of motor.

15. Twist other end of wire to center post of switch

16. Twist red wire from battery holder to second lead of motor.

17. Twist black wire to outer lead of switch.

18. Test circuit with batteries.

19. Place three rubber bands around each can.

20. Attach wheel-axle suspension to chassis by inserting dowel ends into holes of wood on

both ends

21. Secure wheel-axle by wrapping rubber bands on both ends of axle (on outside of wood

base shoe) front and back.

22. Place tape over openings on motor and glue motor to chassis with the stem parallel to

wheel axis and over the “V” Stem facing right makes car move one way, stem facing left

makes car move the other way. Student must test before gluing to determine which way

to glue motor.

23. –Glue battery holder and switch to foam core or wood.

24. –Place rubber band from can around stem of motor (student may have to place holder on

end of stem to keep rubber band from sliding off).

## Wrap-up

End the lesson with some final discussions about what students designed/discovered. Connect their activity back to the focus question/problem, real world examples, and STEM careers.

Also include notes about any special cleanup considerations

# Lesson Background for Teachers

## Suggested Real-World STEM Connections

- List possible related Role Models in STEM (with details and links to more reading). Give suggestions for examples of scientists/engineers who are from underrepresented groups in STEM in related careers.

- List related real-world engineering problems and examples

## Lesson Variations and Options

Include descriptions of alternative methods or ways to present things.

## Explanation

Describe the background concepts and explanations behind the lesson.  This can be more in depth than a teacher/instructor would actually share with students.

## Key Concepts and Vocabulary

- Word 1: Grade Appropriate Definition

- Word 2: Grade Appropriate Definition

## Safety Notes

- Safety instruction