# Speed Boat

| Subject: Physics Related Subjects: Civil Engineer,Electrical Engineers,Electronics Engineers,Hydrologist, Marine Engineers, Naval Engineer | Grade Level(s): Length of Lesson: | Type: Project Keywords: Buoyancy, balance, action-reaction, displacement |
| --- | --- | --- |

# Lesson Overview

Students will create an airboat using plastic containers, motors and propellers.

# Lesson Focus

# Boats have been around almost as long as humans have lived by the sea.  In the late 1700’s, steamboats became among the first motor powered boats. The early 1900’s brought about the airboats, floats with motors elevated above water to create motion by circulating wind.

| # Lesson Objective(s) | By the end of this lesson, students will… - create an airboat, which will float and move in water. |
| --- | --- |

# Lesson Timing

| 5 minutes | Introduce the Lesson |
| --- | --- |
| 30 minutes | ... |

| # Materials | - ¼” wood dowels - Toggle switches - Propellers - 1.5v motors - Insulated wire - Single battery holders - Light plastic container or 1L bottles - Wire strippers - AA batteries - Hot Glue Gun - Glue sticks - Soldering Iron - Solder |
| --- | --- |
| # Instructor Prep | - Step 1 |
| # Related Resources | - List related lessons/ppts etc available from USC or outside, trusted orgs |

# Lesson Plan

## Introduction

Introduce students to the learning goals and make connections to past and present learning experiences. Stimulate interest and prompt students to identify their own questions about the topic. This can link the lesson to real world examples, introduce associated STEM careers, and assess their prior knowledge on the topic.

- Buoyancy, the ability to float, is the upward forces exerted by a fluid on a body in it. If the total area of the object that makes contact with the water is large enough, the object floats. The object must make room for its own volume by pushing aside, or displacing, an equivalent (or equal) volume of liquid. The object is exerting a downward force on the water and the water is therefore exerting a upward force on the object. Of course the floating object's weight comes into play also. The solid body floats when it has displaced just enough water to equal its own original weight.

- Buoyancy is the loss in weight an object seems to undergo when placed in a liquid, as compared to its weight in air. Archimedes' principle states that an object fully or partly immersed in a liquid is buoyed upward by a force equal to the weight of the liquid displaced by that object. From this principle, he concluded that a floating object displaces an amount of liquid equal to its own weight.

- By choosing a plastic body for the boat’s body, we can be sure the boat will be buoyant enough even when we add the motor, propeller, batteries and battery holder.  Students can use a light plastic container (cover of a toy, or headphones package) or tape 2 – 1L bottles together.

- A propeller provides thrust according to Newton's third law, the 'action and reaction' principle: that is, to every action there is an equal and opposite reaction. The propellers accelerate air in one direction, so the boat is subject to a force in the opposite direction.  We mount the propeller so that the air is forced toward the back of the boat, thus pushing the boat forward.

- The propeller must be mounted to the center of gravity of the boat frame so that it remains balanced in the water and doesn’t tip over.

## Procedure

List the steps of the activity.  Remember to focus on leading students to the solution, rather than just telling them what to do.  Include pictures of steps if you have them. Be sure to include some steps for discussion and reflection.

- Strip wires

- Cut a 3 to 5-inch wood dowel (depending on boat frame)

- Glue dowel to center of boat frame

- Insert one wire to one clip on motor

- Insert other wire in other clip on motor

- Solder wires to motor

- Attach propeller to motor

- Test motor with battery to see which end positive battery must touch to make propeller blow air toward you.

- Place small piece of tape on wire which should be connected to positive end of battery

- Solder taped wire to center hole in switch

- Solder red wire from battery holder to either outside hole on switch

- Twist remaining wires and tape

- Glue motor to top of dowel with propeller facing the back of boat frame.

- Glue switch to side

- Glue battery holder to whichever part of boat frame, which maintains balance.

- Place in shallow pool of water.

- Check for stability and balance.

- Flip the switch.

## Wrap-up

End the lesson with some final discussions about what students designed/discovered. Connect their activity back to the focus question/problem, real world examples, and STEM careers.

Also include notes about any special cleanup considerations

# Lesson Background for Teachers

## Suggested Real-World STEM Connections

- List possible related Role Models in STEM (with details and links to more reading). Give suggestions for examples of scientists/engineers who are from underrepresented groups in STEM in related careers.

- List related real-world engineering problems and examples

## Lesson Variations and Options

Include descriptions of alternative methods or ways to present things.

## Explanation

Describe the background concepts and explanations behind the lesson.  This can be more in depth than a teacher/instructor would actually share with students.

## Key Concepts and Vocabulary

- Word 1: Grade Appropriate Definition

- Word 2: Grade Appropriate Definition

## Safety Notes

- Safety instruction