# Steady Hand Circuit

| Subject: Electrical Engineering, Electricity, Circuits Related Subjects: | Grade Level(s): Length of Lesson: | Type: Inquiry / Design / Project Keywords: |
| --- | --- | --- |

# Lesson Overview

Students will create a game, which demonstrates the flow and speed of electricity.

# Lesson Focus

What is the central question or phenomenon being investigated?  What problem are they trying to solve?

| # Lesson Objective(s) | • Students will create a series circuit using a paper clip. • Students will demonstrate the flow of electricity in a closed circuit. • Students will design the circuit to demonstrate hand-steadiness in a game format. |
| --- | --- |

# Lesson Timing

| 5 minutes | Introduce the Lesson |
| --- | --- |
| 30 minutes | ... |

| # Materials | - • LED - • Large paper clips - • 2AA battery holder - • 2AA batteries - • Insulated wire - • Electrical tape - • Foam board - • Hot glue gun and glue sticks - • Scissors or Xacto knife - • Wire strippers |
| --- | --- |
| # Instructor Prep | - Step 1 |
| # Related Resources | - List related lessons/ppts etc available from USC or outside, trusted orgs |

# Lesson Plan

## Introduction

Electricity flows in a series circuit when it has a closed path.

## Procedure

Components and Parts

1. – Cut 5” x 5” foam board

2. – Open one end of a paperclip (see diagram)

3. – Strip the both wires of battery holders

4. – Unwind 2nd paper clip and bend into desired shape

5. – Cut and strip a six-inch wire

Construction

1. – Connect positive end of battery holder to the straight end of the paperclip

2. – Twist the negative (black) wire of battery holder around short lead of LED and tape

3. – Insert ends of bent paperclip into foam core (with 2cm extending out of bottom) and glue

4. – Glue battery holder in place

5. – Twist one end of six-inch wire to the long lead of the LED and tape

6. – Twist other end of wire to bottom of one end of paper clip glued to foam board

7. – Push LED through bottom of foam board and glue

## Wrap-up

End the lesson with some final discussions about what students designed/discovered. Connect their activity back to the focus question/problem, real world examples, and STEM careers.

Also include notes about any special cleanup considerations

# Lesson Background for Teachers

## Suggested Real-World STEM Connections

- List possible related Role Models in STEM (with details and links to more reading). Give suggestions for examples of scientists/engineers who are from underrepresented groups in STEM in related careers.

- List related real-world engineering problems and examples

## Lesson Variations and Options

Include descriptions of alternative methods or ways to present things.

## Explanation

Describe the background concepts and explanations behind the lesson.  This can be more in depth than a teacher/instructor would actually share with students.

## Key Concepts and Vocabulary

- Word 1: Grade Appropriate Definition

- Word 2: Grade Appropriate Definition

## Safety Notes

- Safety instruction