# Lesson Plan Title

| Subject: Electrical Engineering, Mechanical Engineering Related Subjects: Automotive Engineering | Grade Level(s): 7-10 Length of Lesson: 90 minutes | Type: Inquiry / Design / Project Keywords: Electric Motor, Motion, Circuits, Vehicle Design |
| --- | --- | --- |

# Lesson Overview

In this hands-on project, students will design and build a motorized tube car using foam board, cardboard tubes, and a small DC motor. The activity teaches basic principles of mechanics, circuits, and vehicle motion through engineering design.

# Lesson Focus

How can we build a basic vehicle using a motor and simple materials to convert electrical energy into motion?

| # Lesson Objective(s) | - Understand how motors convert electrical energy into mechanical motion. - Assemble a simple electrical circuit to power a moving vehicle. - Apply concepts of torque, friction, and rotational motion. |
| --- | --- |

# Lesson Timing

| 10 minutes | Introduction to electric circuits and motors |
| --- | --- |
| 10 minutes | Material distribution and safety prep |
| 45 minutes | Construction |
| 15 minutes | Testing, troubleshooting, and discussion |

| # Materials | - Foam board (cut to 5”x9”) - Cardboard tube (cut to two 5” pieces) - 4 × 5oz cups - Large dowels (2 × 12”) - Small dowels (2 × 12”) - Electric motor - Rubber bands - 1-AA battery holder - 1 AA battery - Red and black wires - Additional 6” wire - Brass fasteners (2) - Paperclip - Glue stick piece - Tape, glue gun, and scissors - Pencil (for poking holes) |
| --- | --- |
| # Instructor Prep | - Pre-cut foam boards and cardboard tubes. - Prepare motor and battery holders with labeled wires. - Supervise hot glue and cutting stations for safety. |
| # Related Resources |  |

# Lesson Plan

## Introduction

Start with: What makes a car move? How can electricity power a motor? Explain the basics of DC motors and energy conversion. Link to real-world electric vehicles and simple robotics.

## Procedure

- Cut 5” by 9” of foam board

- Cut triangle shape out of foam board 4” from 5” side

- Cut 2 12” pieces from big dowel

- Glue dowels to 5” side of foam board

- Cut 10” cardboard tube in ½

- Poke a hole with pencil through 4 5 oz cups

- Place cups inside cardboard tubes, secure with tape

- Cut 2 12” pieces from small dowel

- Place small dowels through paper cups and cardboard tube

- Glue motor so that stem hangs over triangle shaped edge

- Place rubber band around the stem and cardboard tube

- Secure small dowels across the big dowels with rubber bands

- Poke a hole 2” from edge with big dowel

- Poke another hole 1.75” from 1 st hole

- Place brass fasteners through both holes

- Place a paperclip on first fastener

- Glue 1-AA battery holder to body

- Attach red wire from battery holder to fastener without paperclip

- Attach black wire from battery holder to motor

- Cut 6” wire

- Attach wire from motor to fastener with paperclip

- Cut a small piece from glue stick

- Place small piece on the stem of the motor

## Wrap-up

- What powered the motion of your car?

- What changes affected speed or stability?

- How can we make it go faster or straighter?

Discuss energy transfer, torque, and circuit logic. Link to careers in electrical engineering, automotive design, and robotics.

# Lesson Background for Teachers

This activity combines basic mechanics and circuitry. Students learn that motors convert electrical energy into rotational force, which—via rubber band and axle—propels the car. It mirrors real-world engineering on a simplified scale and builds confidence in hands-on prototyping.

## Suggested Real-World STEM Connections

- Role Models:

Elon Musk – Founder of Tesla and developer of electric vehicles

- Applications:

- Electric vehicles

- Robotics

- Delivery drones

- STEM competitions like FIRST Robotics

## Lesson Variations and Options

- Add a switch to control power.

- Explore gear ratios using pulleys or alternative band placements.

- Challenge students to improve speed or battery efficiency.

## Explanation

Electric motors spin when current flows through their coils, creating a magnetic field. This project uses mechanical advantage (via rubber band and wheels) to convert that spinning into forward motion. Understanding this helps explain many real-world devices from fans to electric scooters.

## Key Concepts and Vocabulary

- Electric Motor: A machine that turns electrical energy into mechanical motion

- Circuit: A closed path through which electricity flows

- Torque: Rotational force

- Friction: Resistance between moving parts

## Safety Notes

- Do not connect battery wires directly together—can overheat.

- Ensure wires are insulated and securely fastened.

- Supervise motor spinning to avoid finger injuries.