# Wiggle Bots

| Subject: Robotics Related Subjects: | Grade Level(s): Length of Lesson: 150 Minutes | Type: Inquiry / Design / Project Keywords: Robot, Power Source, Motor |
| --- | --- | --- |

# Lesson Overview

The motor and battery-pack attached to the cup will cause it to vibrate. When markers are attached, the robot will make interesting markings as it wiggles.

# Lesson Focus

Students hook up a simple DC motor to a battery in order to make a simple, fun robot that wiggles around on a piece of paper and makes its own abstract art.

| # Lesson Objective(s) | - The student will create a functioning robot. |
| --- | --- |

# Lesson Timing

| Time [min] | Description |
| --- | --- |
| 5 | Introduce the Lesson - End Goal - Primer Questions - Key Vocab Words |
| 1 | Role Models in STEM |
| Engineering Method |  |
| 5 | Real World Example |
| 5 | Hypothesis & Prediction |
| 5 | Brainstorm & Design |
| 20 | Build & Create |
| 10 | Test & Record |
| 5 | Assess & Evaluate |
| 5 | How to improve? |
| 10 | Lesson Cleanup |

| # Materials | - Disposable cups - Electrical tape - 3 Markers per student - 1 2-battery holder per student - 2 batteries per student - 1.5-3 V DC Motor per student - Clothespins - Popsicle sticks - Googly eyes (optional) |
| --- | --- |
| # Instructor Prep | - This experiment is more technical than most and requires concise and easy to follow directions. - In particular, the balancing of the wigglebot has proven to be difficult for students in the past, go through this experiment with a methodical approach to keep everyone on track. - Students have had significant difficulty with this lesson, use a visual demonstration approach with verbal explanation of steps as you do them. |
| # Related Resources | - List related lessons/ppts etc available from USC or outside, trusted orgs |

# Lesson Plan

## Introduction

- Name some functions of robots.

  - Toys

  - Washing machines

  - Mars rovers

- What things to robots need to work?

  - Power source

  - 1 or more motors

## Procedure

- Tape the markers inside the cup so that the tips of the markers protrude from the cup; these are the legs. It is important the tips are beyond the edge of the cup.

- To attach the battery holder to the DC motor, wrap the wire from the battery holder around the leads on the motor

- Tape the battery holder (with the DC motor attached) to the top of the disposable cup, slightly off center.

- Tape the motor onto the cup next to the battery holder.

- Place the batteries into the holder to test the motor and the wigglebot’s balance. There should not be too much wiggling at this point. Just make sure there is some vibration but the cup can balance. Adjust placement if needed. This step can potentially be difficult.

- While the motor is still on, attach the clothespin to the top of the motor to make sure it does not cause the cup to fall.

- Turn the motor off and remove the clothespin. The goal is for the cup to wiggle as much as possible without parts falling off.

- Tape a popsicle stick to the clothespin.

- Cut a long, narrow piece of electric tape. Fold the end over the top of the motor. Then, wrap the remainder around the motor so that the sticky side is facing out.

- Attach the clothespin and popsicle stick to the tapewrapped motor. The sticky part of the tape will adhere to the inside of the clothespin and hold it in place.

- Make a face on the wigglebot.

- Take the caps off the markers, plug the batteries in, and let the wigglebot go on a piece of paper.

## Wrap-up

Project Survey - Mack - 03/31/17

The purpose of the survey is to let us know your opinions about today’s project. Remember there are no right or wrong answers.

|  |  | Strongly Disagree | Disagree | Undecided | Agree | Strongly Agree |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | The project increased my interest in science |  |  |  |  |  |
| 2 | After this project, I am more confident in doing science |  |  |  |  |  |

3. What does a robotics engineer do?

4. What two things do all robots need to function?

5. What was the power source of this robot?

Alternate Question

What is a machine?

# Lesson Background for Teachers

## Suggested Real-World STEM Connections

# Real World Example

- Robots are becoming more and more common (drones, self-driving cars, manufacturing robots).

# Hypothesis & Prediction

- How will the placement of the motor and battery pack affect the function of the robot?

# Brainstorm & Design

- How will adding more markers affect the robot?

- What if the lengths of the legs were changed?

# Build & Create

- Students will build wigglebots using common materials.

# Test & Record

- Students will allow their robots to create designs on pieces of paper.

- Students will evaluate the markings made using qualitative observation.

# Assess & Evaluate

- Students will evaluate the success of their robot by assessing its stability and function.

# How to Improve?

What other materials would you use? How could you make the robot wiggle even more?

# Role Model in STEM

Name: Jose M. Hernandez

Years: 1962-Present

Quick Facts

- NASA Astronaut

- Started out as engineer

- Helped invent a digital imaging system to detect breast cancer

- First person to use Spanish language in space

Source(s):

https://en.wikipedia.org/wiki/Jos%C3%A9_M._Hern%C3%A1ndez

## Lesson Variations and Options

Include descriptions of alternative methods or ways to present things.

## Explanation

The motor and battery-pack attached to the cup will cause it to vibrate. When markers are attached, the robot will make interesting markings as it wiggles.

## Key Concepts and Vocabulary

- Motor

  - A machine that supplies motive power to vehicles or devices with moving parts.

- Power Source

  - Hardware component that supplies power to an electrical device (in this case, batteries)

- Robot

  - A machine capable of carrying out a complex series of actions automatically, especially one programmable by a computer.

- Machine

  - A device which can precisely repeat predetermined motions.

## Safety Notes

- Safety instruction