# Wind Vane

| Subject: Mathematics, Physics Related Subjects: Weather Systems | Grade Level(s): 3-7 Length of Lesson: 60 minutes | Type: Project Keywords: Wind direction, Weather tools, Air movement, Climate |
| --- | --- | --- |

# Lesson Overview

In this hands-on activity, students will design and build a working wind vane using everyday classroom materials. They will explore how wind direction is measured and the role of weather instruments in meteorology.

# Lesson Focus

How do scientists measure wind direction, and how can we build a simple tool to do the same?

| # Lesson Objective(s) | - Construct a functioning wind vane. - Understand how wind direction is measured and recorded. - Connect the importance of wind data to weather forecasting and engineering design. |
| --- | --- |

# Lesson Timing

| 5 minutes | Introduction to wind and weather instruments |
| --- | --- |
| 10 minutes | Materials overview and demonstration |
| 30 minutes | Construction and testing |
| 15 minutes | Wrap-up and discussion |

| # Materials | - 1 straw - 1 7oz cup - 1 5oz cup - 1 pencil - 2 note cards - 1 scissors - 1 push pin - 1 ruler |
| --- | --- |
| # Instructor Prep | - Pre-cut triangles from notecards if working with younger students. - Poke pilot holes in cups for ease of assembly. - Review cardinal directions (N, E, S, W) to support wind tracking. |
| # Related Resources |  |

# Lesson Plan

## Introduction

Ask students:

- What is wind? How do we know which way it’s blowing?

- Have you seen weathervanes before? Where? What do they do?

- Show examples or videos of real wind vanes and weather stations. Explain how wind direction is used in weather forecasting and aviation.

## Procedure

- Make a whole through the middle of your 7oz cup, the size of the pencil

- Make another whole size as the pencil to the 5oz cup

- Place the pencil in the middle of both cups attaching them together

- Cut the straw edge 1/2 cm side top and bottom and side left and right (the straw should be straight)

- Cut a triangle 2-sides 4’1/2 cm and bottom side 4cm

- Cut another triangle 2-sides 9cm and bottom side 7’1/2 cm

- Insert the two triangles on the edges of the straw openings, the small triangle pointy side out and the big pointy side in the opening of the straw in

- Put the push pin in the middle of the straw

- Last, put the straw with the wings and the push pin through the middle of the pencil eraser

- Blow!!! It needs to move with the wind.

## Wrap-up

- Ask: What direction did your vane point when you blew on it?

- How would this help a scientist or pilot?

- What other weather tools do you know of?

Discuss the importance of measuring wind for weather prediction, agriculture, and sailing. Introduce STEM careers like meteorologists and climate scientists.

# Lesson Background for Teachers

Wind vanes (or weather vanes) show wind direction by pointing the arrow opposite to the direction the wind is coming from. Understanding wind direction helps scientists predict weather patterns, especially when combined with tools like anemometers (wind speed). This lesson connects physical construction to Earth science concepts.

## Suggested Real-World STEM Connections

- Applications:

  - Weather prediction

  - Airport wind measurements

  - Wind energy site planning

## Lesson Variations and Options

- Label the base with cardinal directions (N, S, E, W) to practice compass use.

- Use a compass app to determine north and align the vane correctly.

- Track wind direction over several days and graph the results.

## Explanation

The wind vane operates on principles of balance and resistance. The larger surface area (tail) catches the wind, causing the pointer (smaller triangle) to face into the wind. This demonstrates basic aerodynamics and real-world engineering in a simple device.

## Key Concepts and Vocabulary

- Wind Direction: The direction from which the wind is blowing

- Weather Vane: A tool used to show wind direction

- Meteorology: The study of Earth’s atmosphere and weather

## Safety Notes

- Use caution when handling scissors and push pins.